STANDARD MICROSYSTEMS CORPORATION

SMC EtherCard PLUS Family LAN Adapter Drivers

For Use in DEC Pathworks Environment

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

INSTALLATION INSTRUCTIONS

-------------------------

1. Insert the DEC PathWorks Client Setup diskette #1 into your

diskette drive.

2. Type: netsetup.

3. Unless you have an LK250 keyboard, specify N(o).

4. Select the DECNET option and press Enter.

5. Insert the DNET TRNSPRT diskette into the diskette drive.

6. Specify the diskette drive letter (A:\ or B:\).

7. Select N(o).

8. Type in the node name and address.  Press Enter.

9. Type in the server node name and number, if required.

If a server already exists on this network, these two

fields are already filled out.

10. Select "Ethernet" and press Enter.

11. Select "LAN Manager-Supplied NDIS."

12. Select "Western Digital EtherCard - WD8003E" option.

13. Press Enter.

14. Modify the PROTOCOL.INI file as required. Make sure

all the values match the assigned values for the

installed adapter. When you are finished, press Enter.

15. Follow the prompts that follow.

16. At the Configuration screen, press F10 to exit.

17. Copy the SMCMAC.DOS file from the \NDIS directory on

your SuperDisk to the \DECNET directory on your hard

disk.

18. Using a text editor, edit the CONFIG.SYS file by replacing

this line:

device=\DECNET\MACWD.DOS

with this line:

device=\DECNET\SMCMAC.DOS

19. Edit the PROTOCOL.INI file by following these steps:

a. Add these lines:

[SMC]

Drivername = SMCMAC$

b. Change the driver names in the [IPX4MAC]

and [DATALINK] bindings statements from

MACWD.DOS

to

SMC

c. Remove the semicolon (;) from in front of the

NI_IRQ statement. Make sure the irq value in

the new [SMC] statement matches the ni_irq value

in the [DATALINK] statement.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their respective

companies.