SECTION 2.0 GETTING STARTED

To use Flipper, you need a synthesizer.  Before you can

start using Flipper, you need to set up one of the supported

synthesizers, and then tell Flipper which kind you are using.

In most cases, this will consist of setting up the synthesizer,

and then running Flipper's "INSTALL" program to set up

the batch file that you need to load Flipper.  If you prefer

to write the batch file yourself, you should have "INSTALL"

write an example batch file and then modify it or use its

contents to write your own batch file.  A plain text editor,

called "EDDIE", is included with Flipper and is useful for

writing and modifying these batch files.

After you have installed your copy of Flipper, you will need

to call Omnichron to get your copy unlocked.  Until it is

unlocked, it will make an announcement every time you

enter the review mode after twenty minutes.  In general,

Flipper will need to be unlocked on each machine that it

will be used on.

The instructions for installing the internal cards are included

with the synthesizers, but we suggest that you either have

your computer store or an experienced friend do the

installation if you have not installed cards before.  The

installation, however, is quite straightforward.  Internal cards

require software to be loaded, either in a "CONFIG.SYS" file

or in a batch file, as explained in their instructions.

The "INSTALL" program which is included on the

distribution disk will automatically configure Flipper for use

on your computer. Before you do the installation, if you will

be running Flipper from a floppy disk, you will need to

format it with the command:

FORMAT B: /s

where the B means the B floppy disk drive.  Hard disks

should be set up to boot MS-DOS before installing Flipper.

To run INSTALL, put the installation disk into your floppy

drive and type INSTALL followed by a return.  Follow the

instructions with a reader, and Flipper will be installed on

your system.

SPECIAL INSTRUCTIONS FOR SERIAL SYNTHESIZER

INSTALLATION:

Serial synthesizers need a working two way connection.  If

Flipper can communicate with the synthesizer, but the

synthesizer can not send characters back, Flipper will only

speak for a fragment of a sentence at a time, and then wait

for the synthesizer.  This can be caused by a bad serial

cable, a bad serial card, or more commonly, by two or more

serial cards or other devices set to the same interrupt or

port number.

DECTALK:

The Dectalk should be connected using the cable which is

included with the Dectalk.  The cable should be connected

to the port labeled COM on the back of the Dectalk.  The

factory setting for the Dectalk is 1200 Baud, and if it is not

changed, you should tell the install program to set up for

that speed.  Setting the Dectalk to higher baud rates will not

significantly increase its response speed.

THE AUDAPTER:

The Audapter should be set to its factory default, which is

9600 baud, one stop bit, no parity, 7 bit characters, and it

should be used with the supplied serial cable.  Flipper can

be set to use the Audapter on COM3 or COM4, if they are

supported by your BIOS.  On PC and AT compatibles, this

will usually require separate software.

THE ACCENT SA:

The Accent SA uses a straight through cable, and used at its

factory default settings, which are 9600 baud, even parity,

one stop bit, 7 bits per character and software (Xon, Xoff)

handshaking. Flipper can be set to use the Accent SA on

COM3 or COM4, if they are supported by your BIOS.  On

PC and AT compatibles, this will usually require separate

software.

OTHER SERIAL SYNTHESIZERS:

In general, Flipper can be used with the serial cables

included with these voice products.  The only connection the

PC needs is a three wire, null modem connection.  However,

several of the synthesizers need handshaking wires which are

included in their factory supplied cables.  These synthesizers

should be set to 9600 baud, even parity, one stop bit, 7 bits

per character and software (Xon, Xoff) handshaking.

The command line used to start Flipper, which is

usually created using INSTALL, includes the following:

Device                                  Command line

Accent Mini or PC or MC                 Flipper M

Accent SA                               Flipper M<serial port>

Artic 200                               Flipper

Artic 210                               Flipper l

Artic 220                               Flipper

Artic 220 /S2                           Flipper l

ASP                                     Flipper @

Audapter from Personal

Data Systems                            Flipper P<serial port>

Braille and Speak                       Flipper B<serial port>

Dectalk                                 Flipper D<serial port>

Dectalk at 1200 baud                    Flipper D<serial port>t

Echo GP or PC                           Flipper e<serial port>

Keynote PC or Keynote PC+               Flipper l

Kurzweil                                Flipper D<serial port>

Pocket Braille                          Flipper B<serial port>

Sounding Board                          Flipper S

Speakeasy                               Flipper V<serial port>

Votrax PSS                              Flipper V<serial port>

Votrax Votalker                         Flipper

other porttalk synthesizer              Flipper S

other echo subset synthesizer           Flipper B<serial port>

(<serial port> means a single digit from 1 to 4 immediately

following the synthesizer character with no space in

between)

Other options which can be added to the command line

include:

Braille to parallel device              =<parallel port>

Braille to serial device                +<serial port>

Do not load built in instructions       h

Do not load lex for Artic

type synthesizers                       l

Switch interface for internal

synthesizer to LPT<N>:                  *<port number>