A d l i b   D o o r

-------------------

by Shvat Shaked, VS inc.

Ver. 1.00

25.2.1992

___________________________________________________

samples playing driver for the Adlib and Adlib Gold

Revision 1.1 : 4.6.1992 by Shvat Shaked

This package contains:

ADL.EXE     - The executable

ADL.DOC     - This file

MP.CHG      - Command file for DEBUG. See below.

MP219.BAT   - Batch file for using MP v2.19 with AdLib. See below.

MP219.ADD   - Command file for MP219.BAT. See below.

What it does?

After running, Adlib Door will turn the lower 6 bits of port 0389H

into a door to the Adlib card. This will allow you to play samples

by outputing them to this port. It will also allow you to use some

programs like MODPLAY or SCREAM TRACKER, after making some easy

changes in them.

How to use it?

The sytax of the executable program ADL.EXE is as follows:

> ADL /ON : installs Adlib Door

> ADL /OFF : Unistalls Adlib Door

The second function is not available for UNREGISTERED users.

And technically ...

Adlib Door is not a TSR (Terminate and Stay Resident) program. It

will not take any memory, and it takes only 5Kb on disk. Its only

purpose is to put ten to twelve words into the control ports of

the Adlib card. You certainly think that I could write a 100 bytes

assembly program that does that ... so be sure I couldn't. The

complexity of the timing needed to write something to the Adlib

forced me to use Turbo Pascal to right the program. To play a

sample just take its HIGHER 6 bits and output them into the LOWER

6 bits of port 0389H (905 Decimal). You can output the sample in

any speed.

Using Scream Tracker v2.2

To use ST you have step the following:

1. Make sure you have DEBUG or SYMDEB in your path.

2. Run ST.

3. Choose Other/Setup and set:

Output Device - Covox LPT1

Playing Speed - To fit your computer speed

Enable Timer  - No

4. Choose Save Setup

5. Press Ctrl+D for Dos Shell

6. Check in what segment the ST is (in DOS 5 you can do it by

typing > MEM /P).

7. Type the following:

> ADL /ON

> DEBUG or SYMDEB

Next, type the following by changing seg to the correct segment:

- RCSseg

- A65F1

MOV DX,0389H

SHR AL,1

SHR AL,1

OUT DX,AL

JMP 65A8

<ENTER>

- A65A3

JMP 65F1

<ENTER>

- Q

Back in the DOS prompt, type:

> EXIT

7. Now load a song, a module or an Amiga module and play it.

8. Have Fun!

Using MODPLAY v1.12

The steps needed to use MP contain two parts. The first part has

to be done one time, and will make a program called MP1. The

second part has to be done anytime you want to run the MP.

First part (for one time running!):

1. Make sure you have DEBUG or SYMDEB in your path.

2. From DOS prompt, type:

> DEBUG MP.COM < MP.CHG

This will produce a program called MP1.COM. Any action concerning

the Adlib card must be done on this program, and this program

only!

Second part (everytime you run):

1. From DOS prompt, type:

> ADL /ON

> MP1 /6 /i  (the 'i' must be lowercase!)

2. Choose a module, and play.

Using MODPLAY Pro. v2.19 (and v2.19b)

It is very easy to change the MP v2.19 to run any kind of hardware

product. I'm doing it by adding some assembler statements to the

MODPLAY.CFG (the MP configuration file). The things YOU have to do

follows:

1. Copy the files MP219.BAT and MP219.ADD to your MODPLAY

directory. For example:

> COPY MP219.* C:\MODPLAY

2. In your MODPLAY directory, run the MP219.BAT by typing:

> MP219

3. Don't forget to run ADL.EXE anytime you want to use the adlib.

Because it isn't a TSR program, you can run it through your

AUTOEXEC.BAT.

4. That's it. Now run the MP the way you always did. It will play

the modules through your Adlib. Note that because the new MP

playes in stereo, and I mix the two channels myself, some of the

quality of the sound is lost. Next time buy Adlib-Gold 2000 or

a Sound Blaster...

For example:

> ADL /ON

> MP

And what now ?!

If you find this program fine, fun, or thin, please, register it

by sending a minimal fee of 12 NIS (5 U.S dollars) to the address

below. Registration will make you able to turn off the noisy card

and on the other hand, will make ME more glad, happy, merry,

joyful but not rich. Questions, answers, bug reports, suggestions

and other vegetables should be left for me on FIDONET (NET 401) or

in one of the following BBSs:

The Small BBS   +972-2-634-203

The C.I.A BBS   +972-2-785-038

InterNet address:

lando@brachot.jct.ac.il

(It is not mine, but of a friend. Put my name on the mail).

My address:

Shvat Shaked

Shimoni 54, Jerusalem

92630, Israel

Tel. +972-2-781510

### End of Original Documention ###