MicroSoft LAN Manager installation with NE2000 Compatible NDIS driver

-----------------------------------------------------------------------

for

NE2000 Compatible EtherPair/EtherCoax Family LAN Adapter Driver

For MIcroSoft LAN Manager NDIS Environments

1. Installation Introduction for LAN Manager V2.0

--------------------------------------------------

a. Type Setup In LanManager Directory. Press Enter.

b. Press Enter.

c. Choice "Import Network Drivers..." in "Action". Press Enter.

d. Type the specific source driver. (A:\> or B:\>).

e. Insert Driver Disk in specific drive. (A:\> or B:\>). Press Enter.

Import Network Drivers appear.

f. Press the SPACEBAR to select Network Adapter Card, "NE2000 Compatible

Ethernet Adapter Card" . Press Enter.

g. Press NO button.

h. Press Enter three times.

Network Driver appear.

i. Choice NE2000 Compatible Ethernet Adapter Card in Network Adapter Cards

select menu , then press <Add driver> button, and press <Add protocol>,

press <OK>. Workstation Primary parameters appear,press <OK>.

k. Press <OK> three times.

l. Choice "Save" in "Action".

m. Press F3 to Exit.

*******************************************************************************

2. Installation Introduction for LAN Manager V2.1

--------------------------------------------------

a. Type lmsetup In LanManager Directory. Press Enter.

Microsoft LAN Manger Setup appear.

b. Choice "Network Drivers..." in " Configuration ". Press Enter.

c. Choice <Add New Configs> ,press Enter.

Network Adapter Driver appears.

d. Choice <Other Driver> , press Enter.

e. Insert the Driver Disk to specified drive (A:\> or B:\>),key in the specified

drive (A:\> or B:\>),then push <ok> button.

f. Select a available Network Adapter Driver, NE2000 Compatible

Ethernet Adapter Card. Push <OK> button.

g. Press the SPACEBAR to select the protocol what you want.

h. Push <OK> button.

Configuration Complete appear.

i. Push <Save> button.

j. Press "F3" to exit.

k. Press Enter.

********************************************************************************

Sample configuration files:

3. Config.sys

a.<V2.0>

LASTDRIVE=Z

BUFFERS = 35

FILES = 35

SHELL=C:\DOS50\COMMAND.COM C:\DOS50\e:1000 /p /E:1024

DEVICE=C:\HIMEM.SYS

DEVICE=C:\DOS50\SETVER.EXE

DOS=HIGH,UMB

DEVICE=C:\LANMAN.DOS\DRIVERS\PROTMAN\PROTMAN.DOS /i:C:\LANMAN.DOS

DEVICE=C:\LANMAN.DOS\DRIVERS\ETHNE\ACCNE\ETHNE.DOS

DEVICE=C:\LANMAN.DOS\DRIVERS\PROTOCOL\NETBEUI\NETBEUI.DOS

b.<V2.1>

LASTDRIVE=Z

BUFFERS = 35

FILES = 35

SHELL=C:\DOS50\COMMAND.COM C:\DOS50\e:1000 /p /E:1024

DEVICE=C:\HIMEM.SYS

DEVICE=C:\LANMAN.DOS\DRIVERS\DOSUTILS\EMM386.EXE NoEMS

DEVICE=C:\DOS50\SETVER.EXE

DOS=HIGH,UMB

DEVICE=C:\LANMAN.DOS\DRIVERS\PROTMAN\PROTMAN.DOS /i:C:\LANMAN.DOS

DEVICE=C:\LANMAN.DOS\DRIVERS\ETHNE\ACCNE\ETHNE.DOS

4. Protocol.ini file for LAN Manager.

a. V2.0

[PROTOCOL MANAGER]

DRIVERNAME = PROTMAN$

[NETBEUI_XIF]

Drivername = netbeui$

BINDINGS = "ACCNE_NIF"

[ACCNE_NIF]

; PROTOCOL.INI for NE2000 Ethernet Aadapter

drivername = ETHNE$

;   irq = 3

;   iobase = 0x300

; End of this sample file

b. V2.1

[PROTMAN]

DRIVERNAME = PROTMAN$

DYNAMIC = YES

PRIORITY = NETBEUI

[NETBEUI_XIF]

Drivername = netbeui$

SESSIONS = 6

NCBS = 12

BINDINGS = "ACCNE_NIF"

LANABASE = 0

[ACCNE_NIF]

; PROTOCOL.INI for NE2000 Compatible ethernet adapter

drivername = ETHNE$

;   irq = 3

;   iobase = 0x300

; End of this sample file

5. Autoexec.bat file for LAN Manager.

a. V2.0

@echo off

PROMPT $P$G

PATH=c:\PE2;C:\DOS50;C:\NC;

APPEND=C:\PE2;C:\DOS50;

REM ====== LANMAN 2.0 === DO NOT MODIFY BETWEEN THESE LINES ===== LANMAN 2.0 ====

SET PATH=C:\LANMAN.DOS\NETPROG;%PATH%

C:\LANMAN.DOS\DRIVERS\PROTMAN\NETBIND

NET START WORKSTATION

REM ====== LANMAN 2.0 === DO NOT MODIFY BETWEEN THESE LINES ===== LANMAN 2.0 ====

b. V2.1

@echo off

PROMPT $P$G

PATH=c:\PE2;C:\DOS50;C:\NC;

APPEND=C:\PE2;C:\DOS50;

@REM ====== LANMAN 2.1 === DO NOT MODIFY BETWEEN THESE LINES === LANMAN 2.1 ===

SET PATH=C:\LANMAN.DOS\NETPROG;%PATH%

NET START WORKSTATION

LOAD NETBEUI

@REM ====== LANMAN 2.1 === DO NOT MODIFY BETWEEN THESE LINES === LANMAN 2.1 ===