@QVRemote.exm  Ver 0.9

Toshiki

<toshiki@j.email.ne.jp>

1. Introduction

QV Remote is a remote control software for the QV-10/100

digital camera made by Casio.

This software is a HP200LX port of -- actually a subset of --

QV Play, a program made by Ken-ichi Hayashi and itojun for a

Unix platform.  The source code of QV Play was utilized

extensively for writing this software.  The author would

like to express his sincere thanks to the authors of QV Play

for releasing its source code in such an open manner, as

expressed in its readme file as follows:

The copyright of qvplay (and qvrec and qvalldel) is owned

by Ken-ichi Hayashi, itojun and all the people engaged in

its development.  The modification of its source code and

its reuse is permitted under the condition that the

source code, modified or reused, is openly available with

the program.  Hopefully there should be notification in

its readme file that the source code of qvplay is used.

2. Files

This archive contains following files.

qvremote.exm   Program

qvremote.doc   User's guide in English (this file)

qvremote.jpn   User's guide in Japanese

readme.txt     simple description of the program

readme.jpn     simple description in Japanese

qvremote.icn   icon file

qvremote.c     source code

qvremote.h     source code

bmpjpeg.h      source code

3. Installation

3.1 Connection

(1) If you do not like to touch a soldering iron, use the

following set:

- Casio's connection cable for a PC (Part Number QC-1D)

- Null modem adapter (9 pin to 9 pin) included in the HP

connector set (Part number F1023A)

- HP Connection cable for PC (Part number F1015A)

(2) If you do not want such a bulky combination, you can use:

- Casio's connection cable for a PC (Part Number QC-1D)

- 9 pin to 10 pin adapter (you should make this yourself)

(3) If you are good at hobby electronics, you can make the

entire cable assembly. Please refer to the information on

the home page of Mohyama:

http://www.kiwi-us.com/~mohyama/article/QV10/

or Horimoto:

http://www.bekkoame.or.jp/~horimoto/QV-MAX/

3.2 Program installation

(1) Add QVREMOTE.EXM to System Manager or to MoreEXM. (MoreEXM

is a freeware utility made by Hiroyuki Sekiya.)

(2) Start QVRemote and Press [MENU], O and D (Option ->

Directory).  Enter the storage directory name to hold your

image data.  This directory should be created by Filer or

by DOS MD command, outside of QVRemote.

4. Usage

4.1 Keystrokes

(1) Function keys when started

[F9]  Connect ... Connect to the camera

[F10] Quit    ... Quit from the program

(2) Function keys after connection

[F4]  Goto    ... Jump to the specified frame

[F5]  Protect ... Toggle to set/clear the image protection

[F6]  Disp    ... Toggle to show/hide the frame number

[F7]  Mode    ... Switch to normal/2x2/3x3 screen

[F8]  Status  ... Get status info from the Camera (Switch

position)

[F9]  Disconn ... Disconnect from the Camera

[F10] Snap    ... Snapshot

(3) Following keys are active while the connection is

established.

+   ... Next frame

-   ... Previous frame

DEL ... Delete (Same as the menu View-Delete)

4.2 Menu

(1) Menu structure

+- Save as

+-  File  -+- Run LxPic

|          +- Exit

|

|          +- Normal

|          +- Split in 4

|          +- Split in 9

+-  View  -+- Custom

|          +- Protect

|          +- Move

|          +- Delete

|          +- Other  -- Color bars

|

|          +- Set Default

+-  Action-+- Snap

|          +- Snap+Save

|          +- Power Off

|

|          +- Directory

+-  Option-+- Communication

|          +- Other

|

+-  Quit

|

+-  Help  --- About

(2) Using menus

o File-Save as

Save image file. Picture image is transferred from the

camera and saved under the following file name:

a:\aaaaa\aa\bbbbcccd.eee

where

a:\aaaaa\aa\  Data storage directory specified in

Option menu.

bbbb          Volume name (specify when saving)

ccc           frame number

d             frame type

N Normal resolution

F Fine image (QV-100 only)

T Thumbnail image

eee           JPG for normal and fine image and

BMP for thumbnail image.

(ccc, d, eee is automatically added)

Note: 1) Even when a file with the same name exists,

QVRemote overwrites without warning.

2) When a file transfer is interrupted, a corrupted

file may be present. Check and delete it if

necessary.

o File-Run LxPic

Launch LxPic.  You can select a single picture, all

pictures under the same volume name or all the pictures

within the storage directory. LxPic is a freeware

utility created by Stefan Peichl. You need to use its

V.5.0 or higher to work with QV-10 normal resolution

images.

o File-Exit

Exit from QVRemote.

o View-Normal

Set to normal screen.

o View-Split in 4

Set to 2x2 split screen.

o View-Split in 9

Set to 3x3 split screen.

o View-Custom

Specify the frame numbers in the split screen.  Entry

screen for the current split mode will appear as follows.

[  ][  ]    [  ][  ][  ]

[  ]  or [  ][  ] or [  ][  ][  ]

[  ][  ][  ]

You can enter here as follows:

[1 ][1 ][1 ]

[1 ][1 ][1 ]

[1 ][1 ][1 ]

to make a multiscreen presentation of the same image.

o View-Protect

Set or clear the protection of the current image.

o View-Move

Change the order of the images in the camera, by moving

the current image.  Specify where to move.

o View-Delete

Delete images from the Camera. Chose options of a single

image, all the unprotected images, or all the images.

The last option is a dangerous one.  Use with care.

o View-Other-Color Bars

Shows color bars on the screen.

o Action-Set Default

Specifies the default screen (the frame to be displayed

the next time the camera is switched on).

o Action-Snap

Takes a snapshot. Same as the release button on the

camera. The REC-PLAY switch should be set to REC

position.

o Action-Snap+Save

Takes a snapshot and save the image file.

o Action-Power Off

Switches the camera off.

o Disable Automatic Power Off

Disables the power save function of the camera. Use with

care. This status will be reset to the initial status by

pressing [F9](Disconnect) or [F10](Snapshot) key.

o Option-Directory

Sets the data storage directory.

o Option-Communication

Set the communication speed (baud rate) for file

transfer. Normally set it to 57600 bps.  If you find

too many retries during the file transfer, lower the

baud reate.

o Option-Other

Following options can be set.

- Suppress Connect Sound

- Disable Automatic Power Off (This function is effective

after the next disconnection by a keystroke [F9]

(Disconn) or [F10](Snap) )

- Delete after Snap+Save (Delete the image data in the

camera after Snap+Save).

- Set Snap+Save filename to (File name used by Save+Save

command. Selection from the following 3 options)

o Normal (Same as Save As)

o Time Stamp (saved as MMDDhhmm.JPG)

o Fixed (fixed as SNAP.JPG)

- Save raw image data (Store the data as transferred from

the camera without adding JPEG/JFIF headers.  Do not

mark this checkbox in normal usage.  This is an option

for maintenance)

o Help-About

Displays the information about this Program.

5. Display

Main screen of QVRemote is as follows

+-----------------------------------------------------------+

|  Status:                  Camera ID:                      |

|  Switch Code:             Battery:                        |

|                                                           |

|                                                           |

|                                                           |

|  Used:                                                    |

|  Remain:                                                  |

|  Picture Size:                                            |

|  Current:                                                 |

|                                                           |

+-----------------------------------------------------------+

Status: When the connection is established, the camera type

is detected and a status such as "Connected to QV-10" is

displayed.

Camera ID: ID string from the camera. QVRemote determines the

Camera type from this ID.  If the camera type is

determined incorrectly, the Camera ID is new to QVRemote.

Please inform the Camera ID to the author in such case.

Switch Code: This code from the camera contains the

information about the status of the current switch

settings.  The interpretation of this code is shown in

the following two lines with the following signs.

(NORMAL)(REVERSE) ... Lens orientation

(REC)(PLAY) ......... Switch position

(BRIGHT)(DARK) ...... Warning sign "Too bright" or "Too

dark"

[PROTECT][ZOOM][DISP][MODE][DEL][-][+][::]

... The status of each pushbutton. ([::]

is the release button)

Battery:  Battery voltage of the camera.

Used: The number of frames stored in the camera.

Remain:  The number of remaining frames. (N) is for normal

images and (F) for fine images.  This function works only

for QV-100.

Picture Size: Image file size information (QV-100 only)  See

below for details.

Current:  Frame number and attribute of the current image

with display mode. 'P' is for protection and 'F' is for

fine mode image (QV-100 only).  (1/9) is for the 3x3

split screen and (1/4) for 2x2 split screen.

6. File formats

QVRemote saves a thumbnail image (small image with 52x36

pixels) in BMP format and a normal image (480x240 pixels)

and a fine mode image (640x480 pixels) in JPEG (JFIF) format.

Normal mode images are expanded in horizontal direction. You

need to resize it to 320x240 with a graphic software.  For

viewing such images by LxPic on Palmtop, adjust the 'pixel

aspect ratio' to 1.2, by pressing [F6] then '2'.

The image data in normal and fine mode files is the output

from the camera with no modification.  Touch-up with your

favorite graphic software is needed to optimize brightness,

color and other features of the images.

According to reports, some programs cannot display the JPEG

format used by QV-10 normal mode.  I found that at least the

following software can display the images correctly:

Graphic Workshop

Irfanview32

GV

Netscape

Internet Esplorer

LxPic  (V5.0 or higher)

The original program QVplay has a variety of output file

formats: JPEG(JFIF), PPM, RGB(Q0), CAM, and BMP(24bit).

QVRemote has a limitation in this regard.  Transfer of

uncompressed data is not supported by QVRemote, since it

would take long time and large storage space, not practical

on a palmtop PC.

7. Scheduled action

You can take a picture and save its image file automatically

from AppointmentBook application.  Use a System Macro such

as:

{Alt+F4}{F9}{Alt+A}v{F9}{F10}

and add it to your AppointmentBook.  {Alt+F4} in this

example is a hot key for QVRemote.  You should change it to

the key you assigned for QVRemote.

For repeated recording, follow the procedures as below.

(1) Add the above macro to a function key ([F9] in this

example).

(2) Make a file (AUTO.CSV in this example) in the comma

separated variables(CSV) format such as this.

"||9","1997/10/16","23:32","23:32","","0",""

"||9","1997/10/16","23:34","23:34","","0",""

"||9","1997/10/16","23:36","23:36","","0",""

"||9","1997/10/16","23:38","23:38","","0",""

"||9","1997/10/16","23:40","23:40","","0",""

"||9","1997/10/16","23:42","23:42","","0",""

...

Note: If you assigned a function key other than

[F9], you need to change '9' in "||9" to

its number.

(3) Use apptout.exe as

apptout -x any.adb -i auto.csv -o auto.adb -m

to make AUTO.ADB.  Here ANY.ADB is any appointment

database file.

(4) Open AUTO.ADB from AppointmentBook.  Now all is set.

Note: APPTOUT is a freeware created by Mr.Fukushima. you can

get it from:

ftp://ftp.vector.co.jp/pack/dos/util/machine/hp/db/aptot084.lzh

When you record a lot of pictures, exceeding the flash

memory capacity of the QV-10/100, you can use following

methods.

o add an All-Delete command ({Alt+V}DA{F8}) and renaming

of the volume name ({Alt+F}Axxxx{F9}) to another

function key under System Macro, and execute it

periodically (at least once in 94 times for QV-10).

xxxx here is a new volume name.  Or

o Set "Delete after Snap+Save" option and set the save

file name to "Time stamp".

You need to disable the automatic power-off feature of the

QV-10/100 to keep it powered up (Setting in Option-Other

menu). An AC adapter or other external power source to the

camera is a must in such case.

8. Picture Size

Picture size information (QV-100 only) displays information

on the size of the image data.  For example

39fe - 0003 33c4 0314 029e (normal image)

or

d17b - 0101 0000 0000 d0f3 (fine mode image)

====   ==== ==== ==== ====

A      B    C    D    E

A is a size (in hexadecimal) of the image in raw data format

(as sent from the camera). You can convert it to the size of

JPEG files, by adding 503 bytes (decimal) for a normal image

or 471 bytes for a fine mode image

B had two parts. The first 2 digits is 00 (Normal) or 01 (Fine

mode) the other 2 digit is the number of scans, 03 for normal

image and 01 for fine mode.

In case of a normal image, C, D and E are data size of each

scan, Y, U and V components.  In case of a fine mode image,

E is the data size.  The difference between A and the sum of

C, D, E is always 136 bytes -- 2 quantization tables of 64

bytes each plus 8-byte header. (Actually this 8-byte header

is displayed as B, C, D and E)

9. Limitations

o SNAP (take a snapshot) may not work correctly.  If it

fails, try to connect and press SNAP again.  If you find

which condition leads to such failures, please inform the

author.

o Status (Get camera status) sometimes does not work

correctly.

o The author owns the QV-10 only.  The functions on QV-100

are not fully tested.  Its reports are welcome.

10. Copyright, disclaimers, distribution condition

This software is a free software.  Its copyright is owned by

its author Toshiki <toshiki@j.email.ne.jp> and by Ken-ichi

Hayashi, itojun and other authors of QVplay.

Conditions of the usage of QVRemote are the same as those

for QVplay.  Users should agree on the following conditions:

- The user should be aware that using this software may

cause non-recoverable destruction to the QV-10 or the

image data contained in it.

- The author is not responsible for any damages which may be

caused by using this software.

- The author is not responsible for any damages which may be

caused by the contents of document accompanying this

software.

- The user should not seek support from CASIO, if using this

software causes a failure in the QV-10 or its connection

cable.  Before you contact its customer support for a

repair, first verify the condition with the software

supplied by CASIO.

- This software was developed with its basis on the source

code of QVplay (with QVrec and QValldel), which were

developed by Mr Hayashi, itojun and many other

collaborators. The disclaimers stated above are also

applied to all these original authors.

- This software may be distributed being bundled with a

commercial software.  In such case, the author has no

responsibilities for its support.

- This software, except for its beta test version, can be

freely distributed.  Prior notification is necessary for

the following cases:

o Posting it to a commercial BBS/provider

o Including in CDROM or other media in a commercially

published periodical.

o Bundling it with commercial software.

- The distribution of this software should be made as its

entity.  Not only the program file, but also its source

code should be included.  The archive name should not be

changed, if not absolutely necessary.

11. Revision History

Ver0.1  The first version.  For alpha tester only.

0.2  The first beta test version.  Released on

NIFTY-Serve.

0.3  Following modifications were made:

o Added launching of LxPic.

o Added support for transfer speed more than 9600bps.

o Added disabling automatic shutdown. Thanks to Mr.

Goto who analyzed this feature.

o Other many small modifications.

0.4  o Fixed small (but a lot of) bugs

0.5  o Fixed a bug in the routine of disabling automatic

shutdown.

0.6  o corrected the acquisition of file size (QV-100).

o Connection is now recovered after a snapshot.

o Added Snap+Save

0.7  o Now resets Comm Port with task swaps.

o Modified Automatic Power Off menu.

o Added Option Delete after Snap+Save.

o Added filename options for Snap+Save.

0.8  o Removed the bug, that eoi mark was lacking at the

end of JPEG file.

0.9  Official release.