Subject:Repairing PCJR Mother Bds, 64k cards, and memory problems.

(Updated 02/25/91)

It has been my experience that the majority of dreaded 2 beep errors

as well as the 3 beep error, no beeps and generally the A & B errors,

are attributable to the 64k memory chips on the mother board or the

64k card. To confirm rough location of problem, strip PCJR down to

raw mother board with only the power card in place. Attach only the

monitor. Keyboard can be infra-red or cable.

If the infra-red receiver is removed then the cable MUST be pluged

in or you get the 2 beeps for this reason.

If Jr boots normally then add the removed options one at a time

to pin down the problem.

When problem is confirmed to be the mother board the following

technique can be used in most cases although definitely not all.

Note: This method also works with the 64k card and memory sidecars

but a great deal more care is required.

As a preliminary step, leave power on the failing system

for about 15 minutes. Using a finger check, look for any excessively

hot memory chip. A warmer than usual chip indicates a shorted

condition and the chip needs to be replaced. If all memory chips appear

normal then proceed.

1. For testing purposes, a known good chip of 250 nano-seconds or

better can be used. I.E. 200ns, 150ns etc.

The test chip should of the same memory size as the chip

being tested. That is don't use a 256k chip across a 64k chip.

(When a defective chip is found however, it should be replaced by

a 150 nano-second or better chip of the same size to preserve

the original circuit spec.)

2. Form the leads using a straight edge for a friction fit when

piggy-backed on top of another chip.

3. Locate the eight 64k memory chips on JR's mother board. They are

in the left front corner in line with the power supply card.

4. This procedure is easier if you have or can borrow the older

design 7" power supply card. The 11" gets in the way.

5. With power off (incidentally there are no dangerous voltages in

this area) piggy-back one of the eight chips being careful to

align the sixteen pins as close as possible to those on the

original chip. Make sure that the orientation of pin one, the

notched end etc. match for the piggy-backed chip.

6. For insurance gently hold the piggy-backed chip in position with

one finger and turn on power to PCJR. Wait for indication.

7. If error indication still present, power down and move to next chip.

8. If Jr comes up normally, you know which chip must be replaced.

If after the first pass you don't find a bad chip then repeat the

process on the chance that you might have had a bad connection.

If you suspect more than one chip is at fault, you can carefully

piggyback all eight chips with known good chips and repeat the test.

If JR then works, you can remove one chip at a time to isolate the

chips needing replacement. On one 64k card I found four bad chips

this way. The darlington regulator chip on the 7" power card had

shorted causing a +20 volt spike on the +5 volt line before the

fuse blew. The result was one blown chip on the mother board and

four on the 64k card.

If the 64K memory chips happen to be made by "INMOS" corp I recommend

that they all be changed. These chips; very few JR's should have

them, were disquallified and replacement was mandatory due to poor

reliability.

Another known problem that causes similar symptoms is smeared lands

on the solder side of the mother board. The location is always under

the plug in cards and is caused by the case mother board supports

rubbing the pads. This problem was corrected in the yellow mask newer

level EC mother boards.

If you are confident that your problem is not a memory chip and you

have not reworked the mother board in any way, then your best bet

at this point is to seek a replacement mother board.

It may still be worth while if problems persist and if you are not

interested or able to take the $134.00 or so exchange from Boulder,

to socketize all eight of the chips and replace them with a new set

of chips. No guarantees of course, and I would assume that you would

have swapped all the original socketed JR chips such as the CPU

and the BIOS ROMS with another JR first. Also check with JR vendors

and various parts houses for used or surplus mother boards.

In one other case the problem did not indicate beeps but the color

monitor screen turned green. That turned out to be a bad gate

array chip. There are about Four non-available chips on the mother

board. The gate array chip and the two bios ROM chips. There is also

the Character generator ROM and a Memory control PROM located under the

diskette drive (24S10). The Roms can be copied if you have another PCJR

and an EPROM burner. The PROM can also be copied if you have the proper

equipment to read and burn those kind of chips.  When any of these

are bad the usual recourse is to exchange the board with IBM provided

of course that the board is unaltered.

Another technique I recently discovered while working on a crumlyized

512K sidecar which had several opens due to poor workmanship was to

use a 16 led 5v tester which is comercially available. You could also

make up a two lead tester with an led in series with a current limiting

resistor. All terminals of a working 64K or 256K chip except pins 2 and

16 should light the led. With the two lead tester secure one side to

pin 16 assuming you have a good ground circuit and simply test each

lead of each chip recording the results on a table.

******************************************************************

Ref: JR Keyboard (Non-Chicklet) (white mask mother only)

Have confirmed hard failure JR keyboard (verified by substitution).

Unit is a FRU (norepair-replace only item). Can get replacement

from vendor.

Found schematics in Sams Photofacts (cost = $39.00). Unit works

fine on infrared (w/batteries). Cable is OK.

Symptom is rapid fire mostly incorect characters--cannot enter

intelligent data. Has anyone encountered a similar problem?

(Symptom later turned to totally dead keyboard)

Ref: JR Keyboard --- PROBLEM SOLVED --- Keyboard was not the problem

Thanks to all who answered the call. Several clues were invaluable

1. Low voltage whether battery operated or system through cable

causes problem of rapid fire sometimes correct keys, or dead

keyboard.

2. Adding additional or first joystick also sometimes causes same

symptoms.

3. Measured the special 5 volt regulated supply (originates on

System board off 12 volt supply), which supplies power to

keyboard, joysticks and composite vidio multiplexer transistor.

To measure voltage easily, disconnect phone type connector at

keyboard. Pins 2 and 5 (both outside pins--1 and 6 are unused)

should read approximately 5 volts + or - about a half a volt.

These originate from two separate supplies.

Obtain ground for meter from shell of the audio or vidio jacks

at back of JR.

4. Measured Pin 5 @ 3.2 volts instead of 5 volts. This is at the

marginal end of CMOS requirement and explained why one keyboard

worked and the other did not. Batt op of course unaffected.

5. Replacing VR1 the system board regulator chip LM340T5 corrected

problem. This is an off the shelf +5v Reg chip available from

Radio Shack.

6. A problem of erattic and or drifting joysticks was also solved.

7. Advice---those experiencing either keyboard or joystick

problems would do well to check this voltage.

8. Note: regulator chip (TO-220 case) is located on the system

board and not on the power supply (daughter) card.

It is positioned near the vidio output jack.

9. Note: The final mother board design identified by a yellow

component mask instead of white, changed the source voltage

for the keyboard and joysticks from the regulated 5v circuit

to the unregulated 5v supply. That eliminated the keyboard

problem on those boards.

Another problem noted when the keyboard is operated off batteries

is the tendancy for alkaline batteries to reverse polarity when

not used for a long period of time. Checking with a voltmeter

confirms the problem. I've seen this dozens of times. Best bet is

to replace all four batteries.

******************************************************************

If all else fails, and you are inclined to try to repair the PCJR

mother board, I will try to outline the general proceedure I use.

The most useful and probably manditory tool to obtain is a logic

probe. Available from Radio Shack for from $17.00 to $20.00.

The most difficult to service problems are the no response, no beep

and immediate two beep errors (I will assume that all power supplies

have been checked OK.)

Remove the mother board from its case. Examine solder pads in the

area of the daughter cards on the solder side of the board. There

have been many cases of smeared pads in these areas causing shorts.

For all testing of the mother board, simply lay the board on a non-

conductive surface. Observe all the standard Anti-static precautions

that you can. It is best to work in a high humidity environment.

There are no dangerous voltages anywhere in the PCJR. The maximum

supply coming from the brick is around 30 volts A.C..

Attach only the brick, the monitor, and the keyboard w/cable.

Power up and verify that the problem still exists. Power down

whenever removing chips or when you are not measuring circuits.

The first step is to remove the 8088 CPU chip from its socket.

Incidentally I power the logic probe from the disk drive cable socket

on the power card. The red lead goes to the bottom (+5v) pin and the

black lead goes to either of the two center pins (ground). Do not

connect the probe to the top (+12v) pin.

Measure all 40 pins of the socket using the logic probe. Readings

should be as follows with power on.

Open (no probe indication): pins 2 thru 16 and 23,25,30 and

34 thru 39.

Logic Low: pins 1,17,20,21,22 and 31.

Logic High: pins 18,24,26 thru 29,32,33 and 40.

Logic Pulses: pin 19. (this verifies that Osc circuit is working)

Any pin not matching the above table will require additional circuit

investigation. The most obvious indicators are pin 19--the clock

circuit is not working and pin 18--the interrupt chip is stuck.

The plastic clock chips (8284) had a high failure rate. The ceramic

chip version was much more reliable. If you determine that the

crystal is not oscillating, try reversing the leads. Strangely

enough, that sometimes cures the problem.

If all pins are normal, then follow on schematic to find all the

various buffer chips especially ZM49, ZM57 and ZM66. Compare logic

levels of similar data and address lines exiting the chips looking

for abnormal highs or lows. A fair percentage of the time you will

detect a shorted data or address line. The problem then becomes

one of where could the short possibly be. The only answer I can

give at this point is to divide and conquer using the binary

technique of separating the circuit into two parts and then the

remainder etc. until you identify the shorted component.

If pin 18 of the 8088 socket is stuck (interrupt error) and other

pins of the CPU socket especially pins 2 thru 16 show errors then

usually the 8259 interrupt controller chip is bad. Unsolder the

8259 and with both the 8259 and the 8088 out of the board retest

the 8088 socket pins. Pin 18 should now be logic open. Any other

abnormal pin of the 8088 must be investigated.

Check the removed 8259 and or replace with a known good 8259.

Recheck 8088 socket pins. If pin 18 is still not high then one of the

IRQX (IRQ0 to IRQ7) interrupt lines is probably stuck.

The interrupts 0 thru 7 are in priority order. The highest is 0 and

comes from the 8253-5 timer chip.

That's about as far as I can go at this point.

Any suggestions or additions to the above are quite welcome.

Especially welcome would be tips such as connect pin x to pin y to

verify that watchamacallet network is good or bad.

Any replacement parts work on the mother board should be followed

by clipping any excessive lead length. The case has been coated

with silver paint or it has been tin plated. In other words, it is

conductive.

******************************************************************

The usual problem with parallel printer sidecars is a smeared

land pattern around the top rear mounting hole. The smeared solder

is simply removed. Likelyhood of a repeat problem is slim especially

if the sidecar is not overtightened. As insurance, the plastic

surface which caused the smear should be scrapped clean of solder

residue or a circular paper hole reenforcer can be laid over the

surface.

Hank Kennedy     Tie 678-6366       AUSVMI(HKENNEDY)