STANDARD MICROSYSTEMS CORPORATION

NDIS driver

For EtherCard PLUS Family LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

This installation guide provides the following instructions:

1.  Overview

2.  Installing Two Adapters in the Same System

3.  PROTOCOL.INI Parameters

4.  Error Messages

1.  OVERVIEW

------------

The NDIS driver is designed to work with EtherCard PLUS Family LAN

Adapters. It conforms to the Network Driver Interface Specification(NDIS)

defined by Microsoft/3Com. The driver has been tested with Microsoft LAN

Manager, 3Com 3+Open, IBM LAN Server, IBM PC LAN, DEC Pathworks and other

NDIS compatible network operating systems and protocol stacks.

The driver provides support for these three adapter types:

7  Jumper-Configured

These adapters can only be configured by setting jumpers

on the adapter boards.

WD8013EBT   WD8003E

WD8003EBT   WD8003S

WD8003SH    WD8003WT

7 Software-Configurable

These adapter cards have the ability to be configured using

a software setup program.

WD8003EB     8013EP     8013W

8003EP       8013EPC    8013WC

8003W        8013EW

8003WC       8013EWC

7 Micro Channel

This adapter type works only with the IBM Micro Channel bus

protocol standard.

WD8003ET/A     WD8003E/A

WD8003ST/A     WD8003W/A

8013EP/A

8013WP/A

\NDIS

INSTALL.DOC        Installation Guide (this File)

RELEASE.DOC        Version history and release information

SMCMAC.OS2         OS/2 NDIS MAC driver

SMCMAC.DOS         DOS NDIS MAC driver

SMCMAC2.OS2        OS/2 NDIS MAC driver for second adapter

SMCMAC2.DOS        DOS NDIS MAC driver for second adapter

PROTOCOL.INI       PROTOCOL.INI file sample

SMCDOSMC.NIF       NIF file for Micro Channel adapters

using DOS and IBM LAN Server

SMCOS2MC.NIF       NIF file for Micro Channel adapters

using OS/2 and IBM LAN Server

SMCDOSAT.NIF       NIF file for AT adapters using DOS

and IBM LAN Server

SMCOS2AT.NIF       NIF file for AT adapters using OS/2

and IBM LAN Server

WDDOS.NIF          NIF file for Western Digital EtherCard

PLUS adapters using DOS and IBM LAN

Server

WDOS2.NIF          NIF file for Western Digital EtherCard

PLUS adapters using OS/2 and IBM LAN

Server

SMC.MSG            OS/2 Driver Message File

\MSLANMAN.DOS

DRIVERS\ETHERNET\SMCMAC

PROTOCOL.INI    PROTOCOL.INI file for Microsoft LAN Manager

DRIVERS\NIF

SMCMAC.NIF      NIF file for SMC adapters using DOS and

Microsoft LAN Manager

\MSLANMAN.OS2

DRIVERS\ETHERNET\SMCMAC

PROTOCOL.INI    PROTOCOL.INI file for Microsoft LAN Manager

DRIVERS\NIF

SMCMAC.NIF      NIF file for SMC adapters using OS/2 and

Microsoft LAN Manager

\10NET

AUTOEXEC.BAT       Example AUTOEXEC.BAT for 10NET.

CONFIG.SYS         Example CONFIG.SYS for 10NET.

PROTOCOL.INI       Example PROTOCOL.INI for 10NET.

\DEC_PATH

INSTALL.DOC        Installation instructions for DEC PathWorks.

\FTP

CONFIG.SYS         Example CONFIG.SYS for FTP.

PROTOCOL.INI       Example PROTOCOL.INI for FTP.

\PCNFS

INSTALL.DOC        Installation instructions for PCNFS.

\NET_MAN

CONFIG.SYS         Example CONFIG.SYS for NET Manager.

PROTOCOL.INI       Example PROTOCOL.INI for NET Manager.

\LANSERVE

INSTALL.DOC        Installation instructions for IBM LAN Server.

\PCLAN

INSTALL.DOC        Installation instructions for IBM PC LAN

Program.

Install the adapter card in your PC using the instructions in the

Installation Guide included with the adapter.  Proper installation

can be verified by running the Diagnostic Program under DOS.

NOTE: The Diagnostic Program will not run properly from the OS/2

compatibility box.

2.  INSTALLING TWO ADAPTERS IN THE SAME SYSTEM

----------------------------------------------

This release includes SMCMAC2.OS2 and SMCMAC2.DOS.  These drivers

may be used to install a second adapter card.  When installing the

LAN Manager software, select two Standard Microsystems family

adapters. When LAN Manager setup exits, modify the CONFIG.SYS file

and the PROTOCOL.INI file using this procedure.

1. Modify CONFIG.SYS by changing one of the two SMCMAC.XXX entries

from SMCMAC.XXX to SMCMAC2.XXX.  For example:

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\SMCMAC\SMCMAC.DOS

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\SMCMAC\SMCMAC.DOS

should be changed to:

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\SMCMAC\SMCMAC.DOS

DEVICE=C:\LANMAN\DRIVERS\ETHERNET\SMCMAC\SMCMAC2.DOS

2. Modify PROTOCOL.INI by changing the second adapter entry to

specify the DriverName as SMCMAC2$ and changing all parameter

values so no conflict results.

a. For jumper-configured adapters, use a text editor to

modify the following lines for the second adapter entry:

[SMCMAC2_NIF]

DriverName = SMCMAC2$

IOBase = 0x300

RAMAddress = 0xD800

IRQ = 5

Make sure these parameter values do not conflict with

the first adapter entry, and that they match the settings

of the second adapter.

b. For software-configurable adapters, use a text editor to

modify the following lines for the second adapter entry:

[SMCMAC2_NIF]

DriverName = SMCMAC2$

IOBase = 0x300

Make sure the IOBase value does not conflict with the

value provided for the first adapter, and that it matches

the second adapter setting. Always include an IOBase

statement with the DriverName.

c. For Micro Channel adapters, use a text editor to modify

the following lines for the second adapter entry:

[SMCMAC2_NIF]

DriverName = SMCMAC2$

SlotNumber = 4

Make sure the SlotNumber value does not conflict with the

value provided for the first adapter, and that it matches

the second adapter setting. Always include a SlotNumber

statement with the DriverName.

3. Modify adapter values so that the two adapters do not use the same

values for I/O Base Address, RAM Address and IRQ.

Refer to the adapter's Installation Guide for information about

these parameters.

4. Load the drivers and reboot the system.

3.  PROTOCOL.INI PARAMETERS

---------------------------

Several parameters are typically specified in the SMCMAC section of

the PROTOCOL.INI file. The only parameter that must be specified

is the 'DriverName'. All others have default values supplied

by the driver and can be left out of the PROTOCOL.INI file if the

installed adapter matches the default configuration. Software-

configurable and Micro Channel adapters ignore the values of IRQ,

IOBase and RAMAddress and instead use the values in the POS

registers set up when the card was installed.

Below is a list of parameters and their definitions commonly found

in the PROTOCOL.INI file.  Both default values and possible value

ranges are provided for each parameter.

IRQ     Specifies the interrupt used by the adapter.  The

default is 3.  IRQ3 is used by COM2.  If COM2 is in

use, choose another IRQ value.  The interrupt is

selected via a jumper on some adapters, or by way of

the EZSETUP program supplied with other adapters.

Consult the installation guide for information on how

to determine or modify the interrupt. This parameter

is always a decimal number in the range of 2 - 15.

Not all values are supported by all adapters.

IOBase   Specifies the base I/O address used by the adapter.

The default is 0x280.  The address is selected via a

jumper on some adapters, or by way of the EZSETUP

program supplied with other adapters.  Consult the

Installation Guide for information on how to

determine or modify the address.  This parameter is a

hexadecimal number in the range of 0x200 - 0xFFFF.

Hexadecimal numbers must be preceded by '0x' to

distinguish them from decimal numbers.  Remeber that

not all values are supported by all adapters.

RAMAddress   Specifies the Shared RAM segment address used by the

adapter.  This value is established under software

control by the driver.  The default Shared RAM

segment address is 0xD000.  This address is

recommended unless it conflicts with some other

device in the PC.  Consult the adapter Installation

Guide or print the file SYSMAP.DOC on the SuperDisk

for more information on how to select a shared RAM

address.  The driver uses hexadecimal values in the

range of 0xA000 - 0xFFFF.

SlotNumber   Specifies the number of the slot containing the

adapter card in a Micro Channel machine.  This

parameter is a decimal number in the range of 1 - 10.

MaxRequests   Specifies the number of general request queue

entries. The default is 8.  This parameter is a

decimal number in the range of 6 - 24.

MaxTransmits   Specifies the number of transmit queue entries.  The

default is 12.  This parameter is a decimal number in

the range of 6 - 24.

ReceiveBuffers   Specifies the number of receive buffer queue entries.

The default value is 12.  This parameter is a decimal

number in the range of 6 - 24.

ReceiveBufSize   Specifies the size in bytes of a receive buffer.  The

default value is 256.  This parameter is a decimal

number in the range of 256 - 1536.

ReceiveChains   Specifies the number of receive chain header queue

entries.  The default is 12.  This parameter is a

decimal number in the range of 6 - 24.

4.  ERROR MESSAGES

------------------

No SMC8003 Adapters Specified In Protocol.INI Present !

No adapter is installed.  Install an EtherCard PLUS Family LAN

Adapter.

No PROTOCOL.INI entries found for this driver.  Ensure that a

PROTOCOL.INI entry exists with DriverName = SMCMAC$ (or SMCMAC2$).

Checksum of IOBase is incorrect.  Cycle power on the system.

Try configuring the adapter to an alternate IOBase.  If the

problem persists,  consult your adapter vendor.

Protocol.INI parameters do not match the adapter configuration.

POS data or AT bus software configured adapter data does not match

the PROTOCOL.INI entry.  Edit the PROTOCOL.INI file, adjusting the

parameter values to correspond to the adapter configuration.

Conflicting SMC8003 Adapter Parameters Specified In Protocol.INI !

More than one PROTOCOL.INI entry is found for this adapter.  Ensure

that DriverName = SMCMAC$ (or SMCMAC2$) appears only once in

PROTOCOL.INI.

Unknown SMC8003 Family Board ID !

The adapter installed is not supported by this driver.

Consult your adapter vendor for more information.

A parameter value in the Protocol.INI file is out of range.

One of the parameters in the PROTOCOL.INI file entry for this

adapter is out of range.  Consult the INSTALL.DOC file you received

with the driver for valid parameter values.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their respective

companies.