================================================================

EEEEEEE  MM   MM  IIIIIII  L       Y     Y         CCCC   FFFFFF

E        M M M M     I     L        Y   Y         C    C  F

E        M  M  M     I     L         Y Y          C       F

EEEE     M     M     I     L          Y     ---   C       FFFF

E        M     M     I     L          Y           C       F

E        M     M     I     L          Y           C    C  F

EEEEEEE  M     M  IIIIIII  LLLLLLL    Y            CCCC   F

================================================================

A

PC Based Simulator

for the

C-FLEA Virtual Processor

Release 1.0

Revised 09-Feb-94

Copyright 1991-1994 Dave Dunfield

All rights reserved

EMILY-CF: A PC based C-FLEA simulator                            Page: 1

1. INTRODUCTION

EMILY-CF is a software simulator for the C-FLEA Virtual Processor.

It provides a powerful debugging environment, offering many features:

- Supports full 64K of MEMORY. Any portion of memory can be used  for

CODE or DATA.

- Hardware emulation mode communciates  with  a  target  system,  and

allows you to include the physical I/O lines, timers etc.  in  your

simulation.

- Single step, Multi Step, Animate and Full Speed execution modes.

- Simultanious on-screen displays of program disassembly, memory  CPU

registers and simulation messages.

- Full screen editors for CPU registers, I/O registers, a two  memory

editors (CODE and DATA). All of the above may be viewed/altered  at

any time during the debugging session.

- Multiple breakpoints are transparent to the user program.

1.1 System Requirements

EMILY-CF will run on any IBM PC/AT/PS2 compatible system with a

MDA/CGA/EGA or VGA display, and at least 128K of free system RAM.

- If physical serial I/O is used, a  COMM  port  is  required.  If

hardware emulation is also used, TWO COMM ports are required.

- Some CGA displays may exibit "snow" or flicker during high speed

screen updates. This effect will not occur with MDA, EGA or  VGA

displays.

EMILY-CF: A PC based C-FLEA simulator                            Page: 2

2. STARTING UP EMILY-CF

If you are using the optional hardware emulation mode, insure that

the resident control software (supplied) is inserted into the  target

system, and that the PC running EMILY-CF is connected to  the  target

systems serial port  via  COM2.  Power  on  the  test  system  before

starting EMILY. The resident control software  must  be  active  when

EMILY "looks" for it.

EMILY-CF is invoked by typing EMCF at the DOS  command  line.  You

will be greeted with a "welcome" screen which displays a reminder  of

the command line syntax and the copyright message. This  screen  will

remain on the display for about 5 seconds (10  seconds  in  the  DEMO

versin). Pressing any key while this screen is  visible  will  bypass

this delay, and proceed directly to the main control panel.

If EMILY-CF is given a file name on the command line, she loads it

in the same manner as if the 'L'oad command had been used from within

the main control panel.

EMILY-CF: A PC based C-FLEA simulator                            Page: 3

EMILY-CF also accepts several option switches on the command line:

/Hardware[:n] - Enables the hardware emulation feature to cause

EMILY to interact with the target  system  when

performing I/O accesses.  This  option  may  be

followed by a  ':'  and  a  numeric value which

defines the BAUD RATE to be used to communicate

with the target system. Default is 9600.

/Mono *       - Inhibits the  color display, even  if  a  color

adapter is present. This option may  be  useful

if your monitor/adapter combination provides an

unpleasent or unusable display for EMILY.

/Ports:n      - Specify number of parallel I/O ports to provide

on the simulated processor.   If not specified,

a default of 4 ports are provided.

/Swapcom      - Exchanges the operation of the two serial ports

on the PC.  When this option is used, COM2 will

be used for serial I/O redirection.

* Note: If some of the EMILY windows appear  to  be  an  un-readable

washed-out yellow color, try adjusting the color contrast on

your monitor. Those windows should be an easy to read brown.

EMILY-CF: A PC based C-FLEA simulator                            Page: 4

3. USING EMILY-CF

3.1 The Main Control Panel

Once EMILY-CF is up and running, you will be presented with the

"Main Control Panel", which consists of four "windows" displaying:

- A disassembly of program memory       (Upper LEFT)

- The contents of CPU registers         (Upper CENTER)

- The contents of MEMORY                (Upper RIGHT)

- Informational messages from EMILY     (Lower portion)

All of EMILY'S functions are  accessed  by  command  characters

entered from this screen:

3.1.1 A - Animate execution

This function begins execution of the user program (from the

address in the PC register). As each instruction  is  executed,

EMILY will display the  instruction  being  executed,  the  CPU

registers, and the contents of internal RAM in the  appropriate

windows.

When you enter the 'A' command, EMILY will  prompt  you  for

the number of instructions to animate. Pressing return (without

entering  a  number)  will  cause  the  animation  to   proceed

non-stop. Pressing <escape> will return you to the Main Control

Panel.

The animation will terminate when:

- The requested number of instructions have been stepped.

- A breakpoint is encountered.

- An illegal opcode is encountered.

- The <escape> key is pressed on the PC console.

3.1.2 B - set Breakpoint

This command sets a breakpoint such that  program  execution

under the 'A'nimate, or  'G'o,  commands  will  stop  when  the

breakpoint address is encountered.

Up to 10 breakpoints may be in effect at the same time.

You will be prompted for  the  address  (in  HEX).  Pressing

return (without entering an address) will cause the  breakpoint

to be  set  at  the  current  program  counter  (PC)  location.

Pressing <escape> will return you to the main control panel.

EMILY stores the breakpoint addresses internally in a simple

zero terminated list. This allows  her  to  avoid  searching  a

fixed size table, and helps  speed  up  the  operation  of  the

program, especially when there are few breakpoints.  This  does

however cause the restriction that you cannot set a  breakpoint

at address zero ($0000).

EMILY-CF: A PC based C-FLEA simulator                            Page: 5

3.1.3 C - Change register

You will be prompted for the CPU register to change. Enter a

character from the following list, ESCAPE to abort, or F1 for a

reminder:

A   - A accumulator

I   - INDEX register

S   - Stack pointer

P   - Program counter

1-9 - Parallel I/O port 1-9

After you have selected the register, you will  be  prompted

for the new contents (in HEX).

3.1.4 D - edit Data memory

This command allows you to edit memory using  EMILY's  built

in memory editor. There is  no  functional  difference  between

this and the 'P'rogram memory editor. Having two editors allows

you to work with and edit two areas of memory without having to

keep  re-entering  the  addresses.  See  the  "Memory   Editor"

section.

3.1.5 G - Go: execute program

This  function  begins  high-speed  execution  of  the  user

program (from the address in the PC register).

When you enter the 'G' command, EMILY will  prompt  you  for

the number of instructions to execute. Pressing return (without

entering a number) will cause the program to proceed  non-stop.

Pressing <escape> will return you to the Main Control Panel.

Program execution will terminate when:

- The requested number of instructions have been executed.

- A breakpoint is encountered.

- An illegal opcode is encountered.

- The <escape> key is pressed on the PC console. *

* NOTE: If you are using 'Local' serial I/O mode, ESCAPE offers

you a menu of execution I/O functions. See the F3  command  for

more information.

3.1.6 J - Jump to PC

This command re-positions the disassembly display window (if

necessary) to display the instruction addressed by the  current

CPU Program Counter (PC).

3.1.7 K - Kill all breakpoints

Removes ALL breakpoints after prompting for permission to do

so.

EMILY-CF: A PC based C-FLEA simulator                            Page: 6

3.1.8 L - Load program image

Prompts for the name of a file, and loads  it  into  PROGRAM

memory. The file must  be  in  either  INTEL  or  MOTOROLA  hex

download format.

3.1.9 N - Next disassembly window

Advances the disassembler to display the  following  screen.

Subsequent 'N' commands  will  proceed  to  disassemble  higher

memory addresses.

3.1.10 P - edit Program memory

This command allows you to edit PROGRAM memory using EMILY's

built in memory editor. This editor operates identically to the

'D'ata memory editor. See the "Memory Editor" section.

3.1.11 R - Remove a breakpoint

Prompts for the address, and if a breakpoint is set at  that

address, it is removed. Pressing return  (without  entering  an

address) will remove the  breakpoint  at  the  current  program

counter (PC) location.

3.1.12 T - Traceback viewer

In all execution modes, EMILY keeps track of the  last  4095

instructions executed. By entering the  traceback  viewer,  you

can examine this recording, and determine program  activity  up

to 4095 instructions preceeding the one on which execution last

stopped.

The following command keys are available in the viewer:

Up   - Backup the display 1 instruction.

Down - Advance the display 1 instruction.

PgUp - Backup the display 50 instructions.

PgDn - Advance the display 50 instructions.

Home - Position to the beginning of recording.

End  - Position to the end of recording.

F1   - Display a help summary.

F2   - Prompt for position & move.

F3   - Search backward (in time) for address. *

F4   - Search forward  (in time) for address. *

Esc  - Exit the Traceback viewer.

* If you do not enter an address, EMILY searches for the previous

or next instance of the instruction at your current position.

Only the instruction executed  is  saved  in  the  traceback

viewer, the contents of registers, RAM etc. are not recorded.

The instruction at position 0 in the traceback buffer is the

one to which the current  PC  points,  and  has  not  yet  been

executed.

EMILY-CF: A PC based C-FLEA simulator                            Page: 7

3.1.13 F1 - Help display

Pressing this key displays a summary of the  EMILY  commands

which are available from the Main Control Panel.

3.1.14 F2 - Set Disassembler Address

Prompts for  an  address  (in  HEX),  and  re-positions  the

disassembler window to display instructions beginning  at  that

address.

3.1.15 F3 - Setup Serial Port

Opens a window displaying the (B)audrate,  (P)arity,  (D)ata

bits and (S)top bits which are  currently  configured  for  the

PC's COM1 serial port.

Press B, P, D or S to cycle to the next corresponding entry.

When Baudrate is selected as 'Local', EMILY  will  re-direct

the  programs  serial  accesses  to  the  message  window.  Any

characters which are written  to  the  serial  device  will  be

output in the message window, and any character which are typed

at the keyboard will appear as incomming serial data. When this

mode is active, Pressing ESCAPE in the 'G'o command brings up a

menu of execution options:

End simulation  - Stops the 'G'o command.

Send ESCAPE     - Send ESCAPE through as a charcter.

Zoom window     - Switch to full screen I/O window.

If 'Local' is not selected, EMILY  re-directs  the  programs

serial accesses to the PC's  COM1  serial  port,  however,  for

maximum  flexibility,  the  actual  data  format  used  may  be

configured by this menu.

EMILY allows you to "fine tune" a baud rate,  by  using  the

Up/Down keys. Note that the  available  baud  rate  values  are

determined by the PC COMM  port  hardware,  and  may  not  fall

exactly on the value you wish to use.

Asynchronous communication is specified to work properly  as

long as the device is operating within 5% of the intended  baud

rate.

For example:

Desired baud rate       : 1500

Allowable range (+/- 5%): 1425-1575

Closest PC baud rate    : 1496

In the above example, the baud rate produced is within  0.3%

of  the  desired  value,   and   is   therefore   well   within

specifications.

EMILY-CF: A PC based C-FLEA simulator                            Page: 8

3.1.16 <space> - Single step one instruction

Every time you press the <space> bar, EMILY will execute one

instruction, and update the on-screen displays.

3.1.17 PgDn - Advance RAM display page

Pressing this key advances the memory display window by  one

page (64 bytes).

3.1.18 PgUp - Backup RAM display page

Pressing this key backs up the memory display window by  one

page (64 bytes).

3.1.19 Down - Advance RAM display line

Pressing the "down arrow" key advances  the  memory  display

window by one line (4 bytes).

3.1.20 Up - Backup RAM display line

Pressing the "up arrow" key  backs  up  the  memory  display

window by one line (4 bytes).

EMILY-CF: A PC based C-FLEA simulator                            Page: 9

3.2 The Memory Editor

When you activate the  'D',  or  'P'  commands  from  the  main

control panel, EMILY will invoke its internal "Memory  Editor"  to

access the desired memory area. When you are within  this  editor,

the following commands are available:

3.2.1 F1 - Help display

Pressing this key displays a summary of the  EMILY  commands

which are available from within the Memory Editor.

3.2.2 F2 - Set Edit Address

Prompts for an address (in HEX), and re-positions the editor

display and cursor to edit at that address.

3.2.3 F3 - Toggle Edit Mode

EMILY allows two edit modes, HEX  and  ASCII.  When  in  HEX

mode, your cursor will be positioned in the hexidecimal display

portion of the memory  memory  dump,  allowing  you  to  change

memory by entering a new value as two  hexidecimal  characters.

When in ASCII mode, your cursor will  be  positioned  over  the

ASCII portion of the dump, and  you  can  replace  memory  with

ASCII text typed at the keyboard.

Pressing F3 will toggle between HEX and ASCII mode,  causing

your cursor to switch between the corresponding portion of  the

memory display.

3.2.4 Up - Move up 1 line

Pressing the UP ARROW causes the cursor to move  upwards  on

the screen by one line. This reduces the memory edit address by

16 bytes.

3.2.5 Down - Move down 1 line

Pressing the DOWN ARROW causes the cursor to move  downwards

on the screen by one  line.  This  increases  the  memory  edit

address by 16 bytes.

3.2.6 Left - Move left 1 byte

Pressing the LEFT ARROW causes the cursor  to  move  to  the

left by one byte position. This reduces the memory edit address

by 1 byte.

If the cursor is already at the left hand  margin,  it  will

wrap around to the rightmost position of the previous line.

EMILY-CF: A PC based C-FLEA simulator                            Page: 10

3.2.7 Right - Move right 1 byte

Pressing the RIGHT ARROW causes the cursor to  move  to  the

right by one byte position. This in increases the  memory  edit

address by 1 byte.

If the cursor is already at the right hand margin,  it  will

wrap around to the leftmost position of the following line.

3.2.8 PgUp - Move back 1 page

Pressing the PGUP key causes the editor  to  move  back  one

display page. This reduces  the  memory  edit  address  by  256

bytes.

3.2.9 PgDn - Move forward 1 page

Pressing the PGDN causes the  editor  to  move  forward  one

display page. This increases the memory  edit  address  by  256

bytes.

3.2.10 Home - Move to start of line

Pressing the HOME key causes  the  cursor  to  move  to  the

leftmost position of the line it is currently on. If the cursor

was already in the  leftmost  position,  it  is  moved  to  the

leftmost position of the preceeding line.

3.2.11 End - Move to end of line

Pressing the END key  causes  the  cursor  to  move  to  the

rightmost position of the line  it  is  currently  on.  If  the

cursor was already in the rightmost position, it  is  moved  to

the rightmost position of the following line.

3.2.12 Ctrl-PgUp - Move to start of memory

Pressing the CTRL and  PGUP  keys  re-positions  the  editor

display and cursor to address $0000 at the start of the  memory

block.

3.2.13 Ctrl-PgDn - Move to end of memory

Pressing the CTRL and  PGDN  keys  re=positions  the  editor

display and cursor to the  highest  address  available  in  the

memory block.

EMILY-CF: A PC based C-FLEA simulator                            Page: 11

3.3 Hardware Emulation

EMILY-CF supports a hardware  emulation  mode,  which  provides

access to the physical I/O ports, timers etc. from a target system

in your simulation.

Before using EMILY-CF in this mode, you must download  and  run

the companion  Resident  Control  Program  on  the  target  C-FLEA

virtual processor, before running EMILY.

There is an approximatly 50 MS delay  when  accessing  the  I/O

ports and other physical hardware. This occurs due to  the  serial

activity required to communicate the data to and from  the  target

system.  For  most  applications,  this   delay   is   acceptable,

particularly in  the  stop/start  environment  of  simulation  and

debugging.

When using this  (Hardware  Emulation)  feature,  the  Resident

Control Software should be running in  the  target  system  BEFORE

executing EMILY. Be sure to specify '/H:<speed>' parameter on  the

command line when starting EMILY.

When requesting information from the target system, EMILY has a

short timeout, after which a return value of zero is  assumed.  If

EMILY seems to "hang" briefly betweeen one  command  and  another,

and/or all of your port displays suddenly get reset to  zero,  you

have likely lost (or never  established)  communication  with  the

target system.

EMILY-CF: A PC based C-FLEA simulator                            Page: 12

3.4 Messages

During the course of operation, EMILY may display the following

messages in the message window located in the lower portion of the

screen:

3.4.1 All Used!

You have tried to enter a  breakpoint,  when  all  available

breakpoints are in use. EMILY allows up to 10 code  breakpoints

to be active at any one time.

3.4.2 Animating at XXXX

Indicates that EMILY has  commenced  animated  execution  of

your program, beginning at the address 'XXXX'.

3.4.3 Already set!

Issued to indicate that the address at which you have  tried

to set a breakpoint is already active as a breakpoint.

This message will also  result  of  you  attempt  to  set  a

breakpoint at address zero ($0000).

3.4.4 Breakpoints: [XXXX ...]

This message shows the currently set  breakpoint  addresses,

and is issued whenever any  command  involving  breakpoints  is

performed.

3.4.5 Executing at XXXX

Indicates that EMILY has commenced full  speed  exection  of

your program, beginning at the address 'XXXX'.

3.4.6 FILENAME: Bad checksum

EMILY  has  encountered  a  record  containing  a  incorrect

checksum value while attempting to 'L'oad the named file.

EMILY-CF: A PC based C-FLEA simulator                            Page: 13

3.4.7 FILENAME: Invalid HEX record

EMILY  has  encountered  an  improperly  formatted  download

record while attempting to 'L'oad the named file.

3.4.8 FILENAME: N bytes loaded

The  indicated  number  of  bytes  have  been   successfully

'L'oaded from the named file.

3.4.9 FILENAME: Unable to access

EMILY was unable to open and read the named file.

3.4.10 Stopped at XXXX

Indicates that EMILY has stopped execution of you program at

the indicated address. This message is appended to the  initial

message indicating that execution had commenced.

3.4.11 XXXX: Not found

This message is issued while in the 'T'raceback  viewer,  to

indicate that an address 'XXXX' you have searched for does  not

occur between your current position, and the start/end  of  the

traceback buffer.

EMILY-CF: A PC based C-FLEA simulator

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

1.1 System Requirements                                             1

2. STARTING UP EMILY-CF                                                2

3. USING EMILY-CF                                                      4

3.1 The Main Control Panel                                          4

3.2 The Memory Editor                                               9

3.3 Hardware Emulation                                             11

3.4 Messages                                                       12