DU - Disk Utility for CP/M

Originally designed for use with CBBS over phone lines.

Therefore, DU is not "verbose" - it only shows you what

you explicitly ask for.

Functions supported:

Tnn	Seek to track nn

Snn	Seek to sector nn (must do R then)

Gnn	Position to group nn

Fname	print directory for file "name"

L	Logs in the disk (used if a disk other

than the one DU resides on, is placed

in the system)

Ln	Logs in disk 'n', such as: LA

D	Dump sector, hex + ASCII

A	Dump sector, ASCII only

H	Dump sector, hex only

note all dump commands (D, A, H) may be optionally

followed by a starting and ending address:

D0,7F  is the same as just D

D3,5

A20,3F

CHaddr,val,val,val... change hex in sector

CAaddr,char string... change ASCII in sector

(must W to have changes written to disk)

Example:  CA7,WARD  would change 7-A to "WARD"

Note that the C command echoes the overlaid data for

verification.  If you made a mistake, either patch

it back, or type R to re-read the original sector.

+	advance 1 sector (if below track 2,

this advances to next numerical, if

2 or >, advances based on CP/M's normal

sector scrambling algorithm, i.e. so +

will get the next LOGICAL sector of the file

-	backs up 1 logical sector

+ and - may take a hex amount (+F for ex.)

?	Says what sector, track, and group

are current

R	Reads the sector currently positioned to

into memory.  Note R (Read) is implicit in

the G, +, and - commands, but N-O-T in the

S and T commands (I did it because I was

tired of disk reading after T command before

I had a chance to issue the S command, but

probably should have S automatically read)

W	Write back the current sector (N-O-T-E may

not be used after an F command, as CP/M was

used to find the file in the directory, and

you are not really positioned for a write)

X	Exit back to CP/M (Must press return).  Ctl-c

was too easy to hit over modem lines, so I

decided on 2-byte (X, CR) to exit.

########