%OP%DEOperating System Manifests

%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%CO:A,12,80%%C%%H1%%H2%O p e r a t i n g   S y s t e m   M a n i f e s t s%H2%%H1%

The following definitions have been arranged using the standard definition

files included with the Z88 Assembler Workbench. The files were originally

created from the V2.00 release of the Developer's Notes and arranged into a

logical combination of the various topics the system calls define.

The files have been merged and the layout slightly changed for a more visual

appearance of this section which gives a good overview of related calls and

their corresponding call values.

_______________________________________________________________________________

; Alarm manipulation:

DEFC Gn_Aab  = $6809          ; Allocate alarm block

DEFC Gn_Fab  = $6A09          ; Free alarm block

DEFC Gn_Lab  = $6C09          ; Link alarm block in chain

DEFC Gn_Uab  = $6E09          ; Unlink alarm block

DEFC Gn_Alp  = $7009          ; Process an expired alarm (OZ usage)

; Low level alarm call definitions:

DEFC Os_Alm  = $81            ; Alarm manipulation (OZ usage)

; arguments:

DEFC AH_SUSP = $01        ; suspend alarm

DEFC AH_REV  = $02        ; revive alarms

DEFC AH_RES  = $03        ; reset alarm enable state

DEFC AH_SINC = $04        ; display symbol

DEFC AH_SDEC = $05        ; remove symbol (subject to use count)

DEFC AH_SRES = $06        ; reset symbol

DEFC AH_SET  = $07        ; Set a new alarm:

DEFC AH_CNC  = $08        ; Cancel an alarm:

DEFC AH_DG1  = $09        ; Ding-dong type 1

DEFC AH_DG2  = $0A        ; Ding-dong type 2

DEFC AH_AINC = $0B        ; action count increment

DEFC AH_ADEC = $0C        ; action count decrement

DEFC AH_ARES = $0D        ; action count reset

______________________________________________________________________________

; Character management:

DEFC Gn_Cls  = $6209          ; Classify a character

DEFC Gn_Skc  = $3209          ; Bypass a character in a sequense

DEFC Gn_Skd  = $3409          ; Bypass delimiters in a sequense

DEFC Gn_Skt  = $3609          ; Search for character in a sequense

______________________________________________________________________________

; Director/CLI low level access:

DEFC Dc_Alt = $1A0C           ; Pass an alternative character

DEFC Dc_Bye = $080C           ; Exiting current application (internal)

DEFC Dc_Ent = $0A0C           ; Enter new application (internal)

DEFC Dc_Gen = $200C           ; Screen driver call (internal)

DEFC Dc_Icl = $140C           ; Invoke new CLI

DEFC Dc_In  = $0E0C           ; Read from CLI (internal)

DEFC Dc_Ini = $060E           ; Initialize the Director (internal)

DEFC Dc_Nam = $0C0C           ; Name current application

DEFC Dc_Nq  = $160C           ; Handle Director/CLI enquiries (internal)

DEFC Dc_Out = $100C           ; Write to CLI (internal)

DEFC Dc_Pol = $220C           ; Poll for card usage (internal)

DEFC Dc_Prt = $120C           ; Print to CLI (internal)

DEFC Dc_Rbd = $1C0C           ; Rebind streams

; arguments:

DEFC RB_IN = $00         ; input stream - new source of input

DEFC RB_OUT = $01        ; output stream - new destination for output

DEFC RB_PRT = $02        ; printer stream - new destination for output

DEFC RB_INT = $03        ; input stream T - copy of input

DEFC RB_OPT = $04        ; output stream T - copy of output

DEFC RB_PTT = $05        ; printer stream T -  copy of output

DEFC Dc_Scn = $240C           ; Scan for card usage (internal)

DEFC Dc_Sp  = $180C           ; Handle Director/CLI settings (internal)

DEFC Dc_Xin = $1E0C           ; Examine CLI input (internal)

DEFC Os_Use = $EE06           ; Fetch information about process card usage (internal)

DEFC Os_Bye = $21             ; Application exit.

DEFC Os_Exit= $F606           ; Quit process (internal)

DEFC Os_Ent = $FA06           ; Enter an application (internal)

DEFC Os_Cli = $84             ; CLI interface (internal)

; arguments:

DEFC CL_RIM = $01        ; get raw input

DEFC CL_MBC = $02        ; meta/base to character conversion

DEFC CL_CMB = $03        ; character to meta/base conversion

DEFC CL_INC = $04        ; increment CLI use count

DEFC CL_DEC = $05        ; decrement CLI use count

DEFC CL_RES = $06        ; reset CLI use count

DEFC CL_ACK = $07        ; acknowledge CLI/Escape

DEFC CLM_SH = $01        ; SHIFT key (bit) mask

DEFC CLM_CT = $02        ; DIAMOND (bit) mask

DEFC Os_Dom = $FE06           ; Open director memory (internal)

DEFC Os_Poll= $FC06           ; Poll for an application (internal)

DEFC Os_Stk = $F806           ; Stack file current process (internal)

DEFC Os_Wait= $7E             ; Wait for event (internal)

; arguments:

DEFC WT_ANY = $FF        ; infinite

DEFC Os_Fth = $DE06           ; Free tri-handle (internal)

DEFC Os_Gth = $E206           ; Allocate tri-handle (internal)

DEFC Os_Vth = $E006           ; Verify tri-handle (internal)

; arguments for all three handle calls:

DEFC TH_FILT = $F0       ; verify filter handle

DEFC TH_WMG  = $F1       ; verify wildcard manager handle

DEFC TH_ALM  = $F2       ; verify alarm handle

; Application Type manifests 1 (type byte1 in Application DOR):

DEFC AT_Good = 1    ; (BIT 0) well behaved

DEFC AT_Bad  = 2    ; (BIT 1) not well behaved, not killed on pre-emption

DEFC AT_Ugly = 4    ; (BIT 2) not well behaved, killed on pre-emption

DEFC AT_Popd = 8    ; (BIT 3) popdown (if not set then an application)

DEFC AT_Ones = 16   ; (BIT 4) only one application instance allowed

DEFC AT_Draw = 32   ; (BIT 5) OZ to redraw screen is possible (2K file)

DEFC AT_Film = 64   ; (BIT 6) File Manager Application (internal usage)

DEFC AT_Boot = 128  ; (BIT 7) Application Auto Boot (after soft reset)

; Application Type manifests 2 (type byte2 in Application DOR):

; (Bits 3 - 6 are ignored - should be reset)

DEFC AT2_Cl  = 1    ; (BIT 0) Set Caps Lock on creation entry

DEFC AT2_Icl = 2    ; (BIT 1) Set 'inverted' Caps Lock on creation entry

DEFC AT2_Ie  = 128  ; (BIT 7) Ignore error returns (should be avoided).

_______________________________________________________________________________

; DOR interface:

DEFC OS_Dor = $87             ; DOR interface

; DOR codes when using OS_Dor :

DEFC DR_Get    = $01     ; Get handle for a DOR name (internal use)

DEFC DR_Dup    = $02     ; Duplicate DOR

DEFC DR_Sib    = $03     ; Return brother DOR

DEFC DR_Son    = $04     ; Return child DOR

DEFC DR_Fre    = $05     ; Free (release) DOR handle

DEFC DR_Cre    = $06     ; Create blank DOR

DEFC DR_Del    = $07     ; Delete DOR

DEFC DR_Ins    = $08     ; Insert DOR

DEFC DR_Rd     = $09     ; Read DOR record

DEFC DR_Wr     = $0A     ; Write DOR record

; Major DOR types:

DEFC Dm_Dev    = $81     ; File Device

DEFC Dm_Chd    = $82     ; Character Device

DEFC Dm_Rom    = $83     ; ROM information

; Minor DOR types:

DEFC Dn_Fil    = $11     ; File type

DEFC Dn_Dir    = $12     ; Directory type

DEFC Dn_Apl    = $13     ; Application front DOR type,

DEFC Dn_Del    = $7F     ; Deleted Entry type.

; Record DOR types:

DEFC Dt_Nam    = $4E     ; filename

DEFC Dt_Cre    = $43     ; creation time

DEFC Dt_Upd    = $55     ; update time

DEFC Dt_Ext    = $58     ; Extent (length of file)

DEFC Dt_Atr    = $41     ; Not currently used

DEFC Dt_Hlp    = $48     ; Help type

DEFC Dt_Inf    = $40     ; Information

______________________________________________________________________________

; Error return codes:

; Codes marked with * are fatal errors

DEFC RC_OK     = $00

DEFC RC_Esc    = $01          ; Escape condition (e.g. ESC pressed)

DEFC RC_Time   = $02          ; Timeout

DEFC RC_Unk    = $03          ; Unknown request (parameter in register) *

DEFC RC_Bad    = $04          ; Bad arguments *

DEFC RC_Ms     = $05          ; Bad memory segment specifier *

DEFC RC_Na     = $06          ; Not available  *

DEFC RC_Room   = $07          ; No room

DEFC RC_Hand   = $08          ; Bad handle  *

DEFC RC_Eof    = $09          ; End Of File   (also returned by DOR calls)

DEFC RC_Flf    = $0A          ; Filter Full

DEFC RC_Ovf    = $0B          ; Overflow

DEFC RC_Sntx   = $0C          ; Syntax Error

DEFC RC_Wrap   = $0D          ; Wrap condition met

DEFC RC_Push   = $0E          ; Pushback error, cannot satisfy request

DEFC RC_Err    = $0F          ; Internal Error   *

DEFC RC_Type   = $10          ; Unexpected type  *

DEFC RC_Pre    = $11          ; Cannot pre-empt, or No Room

DEFC RC_Onf    = $12          ; Object (file or directory) not found

DEFC RC_Rp     = $13          ; Read protected

DEFC RC_Wp     = $14          ; Write protected

DEFC RC_Use    = $15          ; In Use

DEFC RC_Fail   = $16          ; General Failure, cannot satisfy request

DEFC RC_Ivf    = $17          ; Invalid filename

DEFC RC_Ftm    = $18          ; File Type Mismatch

DEFC RC_Exis   = $19          ; File already exist

DEFC RC_Addr   = $32          ; Bad address  *

DEFC RC_Size   = $33          ; Bad size  *

DEFC RC_Bank   = $34          ; Bad bank  *

DEFC RC_Frm    = $35          ; Frame error  *

DEFC RC_Par    = $36          ; Parity error

DEFC RC_Dvz    = $46          ; Division by zero

DEFC RC_Tbg    = $47          ; Too big

DEFC RC_Nvr    = $48          ; Negative root

DEFC RC_Lgr    = $49          ; Log range

DEFC RC_Acl    = $4A          ; Accuracy lost

DEFC RC_Exr    = $4B          ; Exponent function range

DEFC RC_Bdn    = $4C          ; Bad number

DEFC RC_Draw   = $66          ; Application pre-empted and screen corrupted

DEFC RC_Quit   = $67          ; Request application to quit  *

DEFC RC_Susp   = $69          ; Suspicion of pre-emption

; Related operating system calls and parameters:

DEFC Os_Esc  = $6F            ; Examine special condition

; parameters in A:

DEFC SC_BIT = $00        ; Test for Escape

DEFC SC_ACK = $01        ; Acknowledge escape and flush input buffer

DEFC SC_SET = $02        ; Set Escape (simulate an Escape condition)

DEFC SC_RES = $03        ; Reset Escape without flushing input buffer

DEFC SC_TST = $04        ; Test if Escape Detect. is Enabled/Disabled

DEFC SC_ENA = $05        ; Enable Escape Detection

DEFC SC_DIS = $06        ; Disable Escape Detection

DEFC Os_Erh = $75             ; Set (install) Error Handler

DEFC Os_Erc = $72             ; Get error context (internal)

DEFC Gn_Err = $4A09           ; Display an interactive error box

DEFC Gn_Esp = $4C09           ; Return pointer to a system error message

%P0%

_______________________________________________________________________________

; File open system call:

DEFC  GN_Opf = $6009          ; open file/stream (or device)

DEFC  OS_Op  = $EA06          ; internal open

; Parameters used with GN_Opf (in A register)

DEFC OP_IN   = $01       ; read only (shared)

DEFC OP_OUT  = $02       ; write only

DEFC OP_UP   = $03       ; open read/write

DEFC OP_MEM  = $04       ; Open memory pool

DEFC OP_DIR  = $05       ; Create directory name (returns DOR handle)

DEFC OP_DOR  = $06       ; Fetch DOR handle

; Various file/stream manipulation calls:

DEFC  Gn_Ren = $6609     ; rename filename

DEFC  Os_Ren = $E406     ; file rename (internal)

DEFC  Gn_Del = $6409     ; delete file

DEFC  Os_Del = $E606     ; delete file (internal)

DEFC  Gn_Cl  = $6209     ; close file/stream

DEFC  Os_Cl  = $E806     ; close file/stream (internal)

DEFC  Os_Gb  = $39       ; get byte from file or device

DEFC  Os_Gbt = $3F       ; - with timeout

DEFC  Os_Pb  = $3C       ; put byte to file or device

DEFC  Os_Pbt = $42       ; - with timeout

DEFC  Os_Ugb = $36       ; unget byte (not implemented).

DEFC  Os_Mv  = $45       ; move bytes between stream and memory

DEFC  Os_Frm = $48       ; File read miscellaneous

DEFC  Os_Fwm = $4B       ; File write miscellaneous

; File status codes, when using OS_Frm or OS_Fwm:

DEFC  FA_PTR = $01       ; Get sequential file pointer

DEFC  FA_EXT = $02       ; Get length (Extent) of sequential file

DEFC  FA_EOF = $03       ; End Of File enquiry

DEFC  FA_BST = $04       ; Buffer status (system use only)

; Various calls to process filenames:

DEFC  GN_Pfs = $5A09     ; parse filename segment

DEFC  GN_Prs = $5809     ; parse filename

DEFC  GN_Fcm = $4E09     ; compress a filename

DEFC  GN_Fex = $5009     ; expand a filename

DEFC  GN_Esa = $5E09     ; read/write filename segments

; Calls on wildcards, searching for files, parsing filenames:

DEFC  GN_Opw = $5209     ; open wildcard handler

DEFC  GN_Wcl = $5409     ; close wildcard handler

DEFC  GN_Wfn = $5609     ; get next filename match from wc.handler

DEFC  GN_Wsm = $5C09     ; match filename segment to wildcard string

; File Eprom Interface:

; (deleted/old files are ignored, they can only be fetched by using

; EPROM file format information)

DEFC  OS_Epr = $F006     ; File Eprom Manipulation Interface

; arguments:

DEFC  EP_Save = $00      ; blow RAM file to EPROM

DEFC  EP_Load = $03      ; load file from EPROM (save to RAM file)

DEFC  EP_Dir  = $15      ; return next filename (starting with first)

; (all files must be read until RC_EOF)

______________________________________________________________________________

; Filter manipulation:

DEFC Gn_Flo  = $2209          ; Open a filter

DEFC Gn_Flc  = $2409          ; Close a filter

DEFC Gn_Flw  = $2609          ; Push character into filter

DEFC Gn_Flr  = $2809          ; Read character from filter

DEFC Gn_Flf  = $2A09          ; Flush filter

DEFC Gn_Fpb  = $2C09          ; Push back character into filter

_______________________________________________________________________________

; Floating point package (FPP):

DEFC FP_BAS = $A2             ; call Floating point package function

; (doesn't work properly)

; function codes:

DEFC FP_AND = $21        ; bitwise AND

DEFC FP_IDV = $24        ; integer division

DEFC FP_EOR = $27        ; bitwise XOR

DEFC FP_MOD = $2A        ; integer remainder

DEFC FP_OR  = $2D        ; bitwise OR

DEFC FP_LEQ = $30        ; less than or equal to

DEFC FP_NEQ = $33        ; not equal to

DEFC FP_GEQ = $36        ; greater than or equal to

DEFC FP_LT  = $39        ; less than

DEFC FP_EQ  = $3C        ; equal

DEFC FP_MUL = $3F        ; multiply

DEFC FP_ADD = $42        ; add

DEFC FP_GT  = $45        ; greater than

DEFC FP_SUB = $48        ; subtract

DEFC FP_PWR = $4B        ; raise to the power

DEFC FP_DIV = $4E        ; divide

DEFC FP_ABS = $51        ; absolute value

DEFC FP_ACS = $54        ; arc cosine

DEFC FP_ASN = $57        ; arc sine

DEFC FP_ATN = $5A        ; arc tangent

DEFC FP_COS = $5D        ; cosine

DEFC FP_DEG = $60        ; radians to degrees

DEFC FP_EXP = $63        ; exponential

DEFC FP_INT = $66        ; integer value

DEFC FP_LN  = $69        ; base e (natural) logarithm

DEFC FP_LOG = $6C        ; base 10 (common) logarithmn

DEFC FP_NOT = $6F        ; bitwise NOT

DEFC FP_RAD = $72        ; degrees to radians

DEFC FP_SGN = $75        ; sign

DEFC FP_SIN = $78        ; sinus

DEFC FP_SQR = $7B        ; square root

DEFC FP_TAN = $7E        ; tangent

DEFC FP_ZER = $81        ; zero

DEFC FP_ONE = $84        ; one

DEFC FP_TRU = $87        ; true (-1)

DEFC FP_PI  = $8A        ; PI

DEFC FP_VAL = $8D        ; numeric value of string

DEFC FP_STR = $90        ; string value of a number

DEFC FP_FIX = $93        ; truncate a float to an integer

DEFC FP_FLT = $96        ; integer to float conversion

DEFC FP_TST = $99        ; test number for zero and sign

DEFC FP_CMP = $9C        ; compare two numbers

DEFC FP_NEG = $9F        ; negate number

______________________________________________________________________________

; Integer manipulation:

DEFC Gn_M16  = $7209          ; Unsigned 16bit multiplication

DEFC Gn_D16  = $7409          ; Unsigned 16bit division

DEFC Gn_M24  = $7609          ; Unsigned 24bit multiplication

DEFC Gn_D24  = $7809          ; Unsigned 24bit division

DEFC Gn_Gdn  = $1009          ; ASCII to integer conversion

DEFC Gn_Pdn  = $1209          ; Integer to ASCII conversion

_____________________________________________________________________________

; Interrupt (vector) calls:

; (performed as CALL OZ_DI or CALL OZ_EI)

DEFC OZ_DI = $0051            ; Disable IM 2 interrupts

DEFC OZ_EI = $0054            ; Enable IM 2 interrupts

_______________________________________________________________________________

; Graphic area (PipeDream map) control:

DEFC Os_Map  = $F406          ; Map control

DEFC MP_WR   = $01       ; write a line to the map

DEFC MP_DEF  = $02       ; define a map using Panel width

DEFC MP_GRA  = $03       ; define a map of specific width

DEFC MP_DEL  = $04       ; delete a map

_______________________________________________________________________________

; Memory allocation, deallocation:

DEFC Os_Mop = $4E             ; Open memory (allocate memory pool)

DEFC Os_Mal = $54             ; Allocate memory

DEFC Os_Axp = $D206           ; Allocate explicit page (internal OZ usage)

DEFC Os_Mfr = $57             ; Free memory

DEFC Os_Mcl = $51             ; Close memory (free memory pool)

; Memory information movement:

DEFC Os_Bde = $DA06           ; Copy bytes to extended address

DEFC Os_Bhl = $DC06           ; Copy bytes from extended address

DEFC Gn_Rbe = $3E09           ; Read byte at extended address

DEFC Gn_Wbe = $4009           ; Write byte to extended address

; Memory comparison:

DEFC Gn_Cme  = $4209          ; compare null-terminated strings

; Memory/bank binding:

DEFC Os_Mgb = $5A             ; Get current binding

DEFC Os_Mpb = $5D             ; Set (Put) new binding

DEFC Os_Bix = $60             ; Bind in extended address (internal)

DEFC Os_Box = $63             ; Restore bindings after OS_BIX (internal)

; Fast code interface for bank switching (A=1)

DEFC Os_Fc  = $8A

; Linked list manipulation calls (often used in combination with Os_Mal):

DEFC Gn_Xin = $4609

DEFC Gn_Xnx = $4409

DEFC Gn_Xdl = $4809

; Memory Masks used by Os_Mop:

DEFC MM_S0 = $00              ; Segment 0 mask

DEFC MM_S1 = $40              ; Segment 1 mask

DEFC MM_S2 = $80              ; Segment 2 mask

DEFC MM_S3 = $C0              ; Segment 3 mask

; additional allocation directives for Os_Mop:

; Source of memory (to be allocated):

DEFC MM_FIX = $02     ; fixed workspace (not subject to swapping)

DEFC MM_SLT = $04     ; explicit slot usage, mask'ed with the following:

DEFC MC_CI = $10 ; internal memory

DEFC MC_C1 = $01 ; card 1

DEFC MC_C2 = $02 ; card 2

DEFC MC_C3 = $03 ; card 3

; Allocation variations:

DEFC MM_EXC = $10     ; exclusive use of bank

DEFC MM_MUL = $20     ; use multiple banks

; Segment specifiers used by Os_Mgb, Os_Mpb:

DEFC MS_S0 = $00              ; Segment 0 specifier

DEFC MS_S1 = $01              ; Segment 1 specifier

DEFC MS_S2 = $02              ; Segment 2 specifier

DEFC MS_S3 = $03              ; Segment 3 specifier

; Low level OZ memory organisation definitions:

; (these manifests represents bit identifiers in a configuration byte that

; is (returned by OS_NQ, NQ_SLT):

DEFC BU_EPR = 1               ; bank is EPROM

DEFC BU_ROM = 2               ; bank is ROM

DEFC BU_WRK = 4               ; work memory

DEFC BU_FIX = 8               ; fixed memory

DEFC BU_RES = 16              ; reserved memory

DEFC BU_APL = 32              ; bank is application RAM

DEFC BU_FRE = 128             ; bank is available RAM

_______________________________________________________________________________

; Miscellaneous OZ call definitions:

DEFC Os_Blp  = $D806          ; Bleep

DEFC Os_Prt  = $24            ; Send character directly to printer filter (low level)

DEFC Os_Isq  = $D006          ; Initialize prefix sequense (internal)

DEFC Os_Wsq  = $CE06          ; Write to prefix sequense (internal)

DEFC Os_Fn   = $7B            ; Miscellaneous OS functions (internal)

DEFC FN_AH = $01         ; allocate handle

DEFC FN_VH = $02         ; verify handle

DEFC FN_FH = $03         ; free handle

_______________________________________________________________________________

; Save / Restore manipulation:

; (screen, application mailbox, Page wait & Application card management)

DEFC Os_Sr  = $6C             ; Save & Restore

; arguments:

DEFC SR_SUS = $01             ; Save user screen

DEFC SR_RUS = $02             ; Restore user screen

DEFC SR_WPD = $03             ; Write parameter data (mailbox)

DEFC SR_RPD = $04             ; Read parameter data (mailbox)

DEFC SR_FUS = $05             ; Free user screen (released, not displayed)

DEFC SR_CRM = $06             ; Remvove card (not implemented)

DEFC SR_CIN = $07             ; Insert card  (not implemented)

DEFC SR_PWT = $08             ; Page wait

DEFC SR_RND = $09             ; Occasionally a random number (system use)

_______________________________________________________________________________

; Screen control:

DEFC Os_Off  = $EC06          ; switch machine (screen) off

DEFC Os_Sci  = $D406          ; alter screen info

; arguments:

DEFC SC_LR0  = $01       ; LORES0 (512 byte granularity)

DEFC SC_LR1  = $02       ; LORES1 (4K granularity)

DEFC SC_HR0  = $03       ; HIRES0 (8K granularity)

DEFC SC_HR1  = $04       ; HIRES1 (2K granularity)

DEFC SC_SBR  = $05       ; screen base (2K granularity)

%P0%

_______________________________________________________________________________

; Low level serial port interface:

DEFC Os_Si   = $8D            ; serial interface (low level)

DEFC SI_HRD  = $00       ; Hard reset the serial port

DEFC SI_SFT  = $03       ; Soft reset the serial port

DEFC SI_INT  = $06       ; Interrupt entry point

DEFC SI_GBT  = $09       ; Get byte from serial port

DEFC SI_PBT  = $0C       ; Put byte to serial port

DEFC SI_ENQ  = $0F       ; Status enquiry

DEFC SI_FTX  = $12       ; Flush Tx (transmit) buffer

DEFC SI_FRX  = $15       ; Flush Rx (receive) buffer

DEFC SI_TMO  = $18       ; Set timeout

_______________________________________________________________________________

; Standard input/output calls

DEFC OS_In   = $2A       ; read a byte from std. input (keyboard)

DEFC OS_Tin  = $2D       ; read a byte from std. input, with timeout

DEFC OS_Out  = $27       ; write a byte to std. output

DEFC OS_Xin  = $30       ; examine input (e.g. check for pre-emption)

DEFC GN_Nln  = $2E09     ; send newline (CR/LF) to std. output

DEFC GN_Sop  = $3A09     ; write string to std. output

DEFC GN_Soe  = $3C09     ; write string from extended address

DEFC GN_Sip  = $3809     ; system input line routine

DEFC OS_Pur  = $33       ; purge keyboard buffer (and reset timeout)

; Standard input key definitions (Returned by OS_IN, OS_TIN, GN_SIP)

;

; The following codes are NOT zero prefixed:

; IN_ESC, IN_ENT, IN_TAB, IN_SPC & IN_DEL

DEFC IN_ESC = $1B

;         ENTER          TAB            DELETE

DEFC IN_ENT  = $0D, IN_TAB  = $09, IN_DEL  = $7F

DEFC IN_SENT = $D1, IN_STAB = $D2, IN_SDEL = $D3  ; SHIFT

DEFC IN_DENT = $C1, IN_DTAB = $C2, IN_DDEL = $C3  ; DIAMOND

DEFC IN_AENT = $B1, IN_ATAB = $B2, IN_ADEL = $B3  ; SQUARE

DEFC IN_SPC  = 32

;         LEFT           RIGHT          DOWN           UP

DEFC IN_LFT  = $FC, IN_RGT  = $FD, IN_DWN  = $FE, IN_UP   = $FF

DEFC IN_SLFT = $F8, IN_SRGT = $F9, IN_SDWN = $FA, IN_SUP  = $FB  ; SHIFT

DEFC IN_DLFT = $F4, IN_DRGT = $F5, IN_DDWN = $F6, IN_DUP  = $F7  ; DIAMOND

DEFC IN_ALFT = $F0, IN_ARGT = $F1, IN_ADWN = $F2, IN_AUP  = $F3  ; SQUARE

; various definitions:

DEFC ESC = 27, LF = 10, CR = 13

; Menu exception codes:

DEFC MU_ENT  = $E1, MU_TAB  = $E2, MU_SPC = $E0, MU_DEL = $E3

; OZ screen driver icons:

DEFC SD_EXSP = $20       ; Exact space

DEFC SD_BLL  = $21       ; Bell

DEFC SD_GRV  = $27       ; Grave accent

DEFC SD_SQUA = $2A       ; Square

DEFC SD_DIAM = $2B       ; Diamond

DEFC SD_SHFT = $2D       ; Shift

DEFC SD_VBAR = $7C       ; Unbroken vertical bar

DEFC SD_ENT = $E1, SD_TAB = $E2, SD_SPC = $E0, SD_DEL = $E3

DEFC SD_ESC = $E4, SD_MNU = $E5, SD_INX = $E6, SD_HLP = $E7

; Outlined icons:

DEFC SD_OLFT = $F0, SD_ORGT = $F1

DEFC SD_OUP  = $F3, SD_ODWN = $F2

; bullet icons:

DEFC SD_BLFT = $F4, SD_BRGT = $F5

DEFC SD_BDWN = $F6, SD_BUP  = $F7

; pointer icons:

DEFC SD_PLFT = $F8, SD_PRGT = $F9

DEFC SD_PDWN = $FA, SD_PUP  = $FB

; Miscellaneous driver codes:

DEFC SD_DTS = $7F        ; Delete toggle settings

DEFC SD_UP  = $FE        ; Scroll screen upwards

DEFC SD_DWN = $FF        ; Scroll screen downwards

_______________________________________________________________________________

; System calls to alter settings in Panel, PrinterEd and other resources:

DEFC OS_Nq   = $66  ; enquire (fetch) parameter

DEFC OS_Sp   = $69  ; specify (set) parameter

; Enquiries, read by OS_Nq:

;         <Symbol>  <Value>   <Meaning>

DEFC NQ_Wbox  = $8300    ; return window information

DEFC NQ_Wcur  = $8303    ; return cursor information

DEFC NQ_Rds   = $8306    ; read text from screen

DEFC NQ_Ain   = $8600    ; application enquiry

DEFC NQ_Khn   = $8603    ; read keyboard handle

DEFC NQ_Shn   = $8606    ; read screen handle

DEFC NQ_Phn   = $8609    ; read printer indirected handle

DEFC NQ_Nhn   = $860C    ; read null handle

DEFC NQ_Wai   = $860F    ; Who am I?

DEFC NQ_Com   = $8612    ; read comms handle

DEFC NQ_Ihn   = $8615    ; read IN handle

DEFC NQ_Ohn   = $8618    ; read OUT handle

DEFC NQ_Rhn   = $861B    ; read direct printer handle

DEFC NQ_Mfs   = $8900    ; read free space information

DEFC NQ_Slt   = $8903    ; read slot type information

DEFC NQ_Dev   = $8C00    ; fetch current device

DEFC NQ_Dir   = $8C03    ; fetch current directory

DEFC NQ_Fnm   = $8C06    ; fetch current filename match string

DEFC NQ_Dmh   = $8C09    ; fetch Director special memory handle

DEFC NQ_Inp   = $8C0C    ; read std. input handle

DEFC NQ_Out   = $8C0F    ; read std. output handle

DEFC NQ_Prt   = $8C12    ; read printer stream handle

DEFC NQ_Tin   = $8C15    ; read input-T handle

DEFC NQ_Tot   = $8C18    ; read output-T handle

DEFC NQ_Tpr   = $8C1B    ; read printer-T stream handle

DEFC NQ_Chn   = $8C1E    ; read comms handle

; Specifies, set by OS_Sp:

; Panel definitions (serial port parameters, timeout, defaults, etc.) :

;    <Symbol>  <Value>   <Meaning>

DEFC PA_Gfi   = $8000    ; Install changed settings

DEFC PA_Mct   = $8001    ; Timeout in minutes

DEFC PA_Rep   = $8002    ; Repeat rate

DEFC PA_Kcl   = $8003    ; Keyclick 'Y' or 'N'

DEFC PA_Snd   = $8004    ; Sound 'Y' or 'N'

DEFC PA_Bad   = $8005    ; Default bad process size in K

DEFC PA_Iov   = $8010    ; Insert/Overtype 'I' or 'O'

DEFC PA_Dat   = $8011    ; Date format 'E' or 'A'

DEFC PA_Map   = $8012    ; PipeDream map 'Y' or 'N'

DEFC PA_Msz   = $8013    ; Map size in pixels

DEFC PA_Dir   = $8014    ; Default directory

DEFC PA_Dev   = $8015    ; Default device

DEFC PA_Txb   = $8016    ; Transmit baud rate - binary

DEFC PA_Rxb   = $8017    ; Receive baud rate - binary

DEFC PA_Xon   = $8018    ; Xon/Xoff 'Y' or 'N'

DEFC PA_Par   = $8019    ; Parity 'O', 'E', 'M', 'S' or 'N'

; PrinterED definitions (highlights $ translation tables) :

DEFC PA_Ptr   = $8020    ; Printer name

DEFC PA_Alf   = $8021    ; Allow Linefeed, 'Y' or 'N'

DEFC PA_Pon   = $8022    ; Printer On sequense

DEFC PA_Pof   = $8023    ; Printer Off sequense

DEFC PA_Eop   = $8024    ; End of page sequense

DEFC PA_Mip   = $8025    ; HMI prefix sequense

DEFC PA_Mis   = $8026    ; HMI suffix sequense

DEFC PA_Mio   = $8027    ; HMI offset sequense

; The format of each highlight sequense is as follows:

;    PA_On<n>       Highlight ON sequense

;    PA_On<n>+1     Highlight OFF sequense

;    PA_On<n>+2     OFF at CR 'Y' or 'N'

;    PA_On<n>+3     Highlight n special character

DEFC PA_On1    = $8028   ; Underline

DEFC PA_On2    = $802C   ; Bold

DEFC PA_On3    = $8030   ; Extended sequense

DEFC PA_On4    = $8034   ; Italics

DEFC PA_On5    = $8038   ; Subscript

DEFC PA_On6    = $803C   ; Superscript

DEFC PA_On7    = $8040   ; Alternate font

DEFC PA_On8    = $8044   ; User Defined

; The format of each translation code sequense is as follows:

;    PA_Tr<n>       Translate from character

;    PA_Tr<n>+1     Translate to sequense

DEFC PA_Tr1    = $8048   ; Row 1, Entry A

DEFC PA_Tr2    = $804A   ;              B

DEFC PA_Tr3    = $804C   ;              C

DEFC PA_Tr4    = $804E   ; Row 2, Entry A

DEFC PA_Tr5    = $8050   ;              B

DEFC PA_Tr6    = $8052   ;              C

DEFC PA_Tr7    = $8054   ; Row 3, Entry A

DEFC PA_Tr8    = $8056   ;              B

DEFC PA_Tr9    = $8058   ;              C

; ISO translations 1 - 28:

DEFC PA_Tr10   = $8080   ; Row 1, Entry 1    (ISO 1)

DEFC PA_Tr11   = $8082   ;              2    (ISO 2)

DEFC PA_Tr12   = $8084   ;              3    (ISO 3)

DEFC PA_Tr13   = $8086   ;              4    ...

DEFC PA_Tr14   = $8088   ;              5

DEFC PA_Tr15   = $808A   ;              6

DEFC PA_Tr16   = $808C   ;              7

DEFC PA_Tr17   = $808E   ; Row 2, Entry 1

DEFC PA_Tr18   = $8090   ;              2

DEFC PA_Tr19   = $8092   ;              3

DEFC PA_Tr20   = $8094   ;              4

DEFC PA_Tr21   = $8096   ;              5

DEFC PA_Tr22   = $8098   ;              6

DEFC PA_Tr23   = $809A   ;              7

DEFC PA_Tr24   = $809C   ; Row 3, Entry 1

DEFC PA_Tr25   = $809E   ;              2

DEFC PA_Tr26   = $80A0   ;              3

DEFC PA_Tr27   = $80A2   ;              4

DEFC PA_Tr28   = $80A4   ;              5

DEFC PA_Tr29   = $80A6   ;              6

DEFC PA_Tr30   = $80A8   ;              7

DEFC PA_Tr31   = $80AA   ; Row 4, Entry 1

DEFC PA_Tr32   = $80AC   ;              2

DEFC PA_Tr33   = $80AE   ;              3

DEFC PA_Tr34   = $80B0   ;              4

DEFC PA_Tr35   = $80B2   ;              5    ...

DEFC PA_Tr36   = $80B4   ;              6    ...

DEFC PA_Tr37   = $80B6   ;              7    (ISO 28)

; File system parameters:

DEFC SP_Dev    = $8C00   ; Set current device

DEFC SP_Dir    = $8C03   ; Set current directory

DEFC SP_Fnm    = $8C06   ; Set current name match

%P0%

_______________________________________________________________________________

; Time & Date manipulation:

DEFC Gn_Gmt = $1A09      ; get (read) machine time in internal format

DEFC Gn_Pmt = $1E09      ; put (set) machine date

DEFC Gn_Msc = $2009      ; Miscellaneous time operations

DEFC Gn_Gtm = $0A09      ; convert ASCII string to internal time format

DEFC Gn_Ptm = $0C09      ; convert internal time to ASCII string

DEFC Gn_Sdo = $0E09      ; convert real time to time to elapse

DEFC Os_Dly = $D606      ; delay a given period

DEFC Gn_Gdt = $0609      ; convert ASCII string to internal date

DEFC Gn_Pdt = $0809      ; convert internal date to ASCII string

DEFC Gn_Die = $1409      ; convert from internal to zoned format

DEFC Gn_Dei = $1609      ; convert from zoned format to internal format

DEFC Gn_Gmd = $1809      ; get current machine date in internal format

DEFC Gn_Pmd = $1C09      ; set machine date

DEFC Os_Ht  = $F206      ; hardware time manipulation (internal OZ usage)

DEFC HT_RES = $01   ; reset hardware timer

DEFC HT_RD  = $02   ; read hardware timer

DEFC HT_MDT = $03   ; read monthy/date/time address

DEFC Os_Ust = $EE06      ; update small timer (internal)

______________________________________________________________________________

; Dynamic token manipulation:

DEFC Os_Wrt  = $CC06     ; Write token

DEFC Os_Wtb  = $CA06     ; Write token base

%P0%

lstoff

; Standard Z88 Operating System manifests:

; The information below is defined in Developers Notes, release 2.00

; (c) Cambridge Computer 1989

lston

%CO:B,12,68%%CO:C,12,56%%CO:D,12,44%%CO:E,12,32%%CO:F,12,20%%CO:G,12,8%