%OP%DEInput and the keyboard decoder

%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%CO:A,14,80%%C%%H1%%H2%I n p u t   a n d   t h e   k e y b o a r d   d e c o d e r%H2%%H1%

Input from the keyboard can be achieved by treating it as a device and using

the file I/O calls. Four calls read from the standard input, which unless it

has been rebound by the CLI, will be the keyboard. These calls are:

OS_IN

OS_TIN

GN_SIP

OS_SR

GN_SIP is described later in this section and OS_SR is discussed in

"Miscellaneous useful routines". OS_IN and OS_TIN return the character read in

register A and Fc = 0 to indicate success. Since these are pre-emptable calls

they may return Fc = 1 and any of RC_SUSP, RC_DRAW, RC_ESC, RC_QUIT and, in the

case of OS_TIN, RC_TIME. These calls usually returns ASCII values, but for

special Z88 keys and menu commands, some other encoding is required. The

solution is that when a special key or menu command occurrs, the OS_IN or

OS_TIN will return a zero. The call is then made again and the value returned

by this second call corresponds to the special key or menu command. The codes

for the special keyboard sequences are shown below, those marked * are not zero

prefixed. Keys marked with ** are internal keycodes and not available

to applications. Finally, note that register pair AF and the alternate

registers are corrupted by OS_IN (as almost all system calls). OS_TIN waits for

keyboard input for as long as the number of centiseconds specified in the

BC(in) register pair, and on exit BC will be the centiseconds of the timeout

remaining, and so BC will have changed as well as AF and the alternate register

set.

%H4%Key%H4%

<SQUARE>

<DIAMOND>

<SHIFT>

<LOCK>

<SPACE>

<ENTER>

<TAB>

<DEL>

<ESC>

<MENU>

<INDEX>

<HELP>

<LEFT>

<RIGHT>

<DOWN>

<UP>

***) This 'key' is the forces Lock out state produced by switching the machine

on, and while holding both <SHIFT> keys down, pressing the <CAPS> lock key.

Note that the diamond key operates like a <CTRL> key on a conventional keyboard

and can be used to generate the sub-32 control characters, eg. <>G for BEL,

provided the key in question is not a menu command. The following is a list of

obtainable control characters:

%P0%

%H4%HEX%H4%

$00

$01

$02

$03

$04

$05

$06

$07

$08

$09

$0A

$0B

$0C

$0D

$0E

$0F

$10

$11

$12

$13

$14

$15

$16

$17

$18

$19

$1A

$1B

$1C

$1D

$1E

$1F

%P0%

%H2%The Input Line Routine%H2%

GN_SIP is the standard system input line routine and used by most of the

applications (Diary uses it for editing individual lines on a date page). It

provides access to all the standard editing commands which are:

<>DEL

<>D

<>G

<>M

<>S

<>T

<>U

<>V

<><LEFT>

<><RIGHT>

<SHIFT><LEFT>

<SHIFT><RIGHT>

Where appropriate the following editing commands should be implemented (this is

machine conventions):

<>J

<><UP>

<><DOWN>

<SHIFT><UP>

<SHIFT><DOWN>

<TAB>

<SHIFT><TAB>

The basic specifications of GN_SIP is as follows:

______________________________________________________________________________

GN_SIP, system input line routine

RST 20H, DEFW $3909

IN:

OUT, if call successful:

OUT, if call failed:

Registers changed after return:

______________________________________________________________________________

If bit 0 of register A is set then DE should point to a null-terminated string

which is no greater than the buffer length specified in B. GN_SIP will write

this string to standard output starting at the cursor position and then place

the cursor at the position in the buffer indicated by C. If C is greater than

B, then the cursor is placed at the end of the buffer. Note that if GN_SIP is

suspended then to continue editing the cursor needs to be repositioned to the

start of the input line and GN_SIP called again with A0 set and the buffer

length reset to its initial value. Unless this is done suspension will not be

transparent, eg. whenever <DIAMOND> is inadvertently pressed during input, the

input would be terminated or ots output would be corrupted.

Setting A1 allows you to force insert or overtype mode, with A2, as the mode to

be used in the input line, rather than using the default as set by the Panel.

This can be useful in its own right and in addition can be used to implement a

local insert or overtype mode.

If A3 is not set then input is terminated only by <ENTER>, A = IN_ENT, or

<ESC>. <ESC> will return with A = IN_ESC if escape detection is disabled, or

A = RC_ESC and Fc = 1 if escape detection is enabled. However, if A3 is set

then many more key sequences will cause the routine to exit. All the single

letter Diamond codes will return their control character, except for <>D, <>G,

<>S, <>T, <>U and <>V, which carry out their normal editing functions. Menu

commands in the form of diamond sequences, excepting those starting with D, G,

S, T, U and V, will return their command code. <>S and the <LEFT> and <RIGHT>

arrows, with or without <SHIFT> or <DIAMOND>, will return their codes if an

attempt is made to move outside of the buffer and A4 is not set. Any other key

sequence which has no meaning in the input line routine will cause an exit, eg.

<SHIFT><UP> will return with A = IN_SUP. Note that if A7 is set then <>V will

cause an exit, thus allowing a local inseert/overtype mode to be established,

as is done in PipeDream.

If A4 is set then an attempt to move outside the limits of the buffer will

cause an exit with A = RC_WRAP, and the key which caused the wrapping will be

lost.

A5 is set to select single line lock mode. This sets up a horizontal window of

length L and will scroll the input buffer, if necessary, within this window.

This feature is useful if you have a limited space to display the input line,

but may need a long input string.

%H2%Example%H2%

The following example uses a locked line width of 15 characters. When <ENTER>

is pressed the full input line is displayed. When the buffer is nearly full,

try suspending GN_SIP, eg. by pressing the Square key twice, and you will see a

slight flicker as GN_SIP re-displays and scrolls the line.

_____________________________________________________________________________

include ":*//stdio.def"

include ":*//errors.def"

defc init = 32

; assume that on entry IY points to 40 bytes of free space

; code starts here...

.main

; actual input line subroutine

.fetch

.sip

; define constant string

.pos

_____________________________________________________________________________

%P0%

%CO:B,17,68%

read a character from the standard input

read a character from the standard input, with timeout

standard input line routine

reads a character when doing a "Page Wait". Returns always ASCII 8

or an RC_ error code.

%H4%normal%H4%

IN_SQU, $B8 **

IN_DIA, $C8 **

IN_SHI, $D8 **

IN_LOCK, $E8 ***

IN_SPC, $20

IN_ENT, $0D *

IN_TAB, $09 *

IN_DEL, $7F *

IN_ESC, $1B *

IN_MEN, $E5 **

IN_IDX, $E6 **

IN_HLP, $E7 **

IN_LFT, $FC

IN_RGT, $FD

IN_DWN, $FE

IN_UP, $FF

%H4%DECIMAL%H4%

0

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

DE = buffer for input string

A0 = 1, buffer already contains data to be edited

A1 = 1, force insert/overwrite mode (see A2)

A2 = 1, if A1 = 1, 0 = insert mode, 1 = overwrite mode

A3 = 1, return unexpected characters

A4 = 1, return if wrap occurs

A5 = 1, single line lock control

A6 = 1, disply in reverse video

A7 = 1, if A3 = 1, allow for insert/overwrite return

B = length of buffer

C = cursor position, if A0 = 1

L = width of line, if A5 = 1 (incl. null-terminator)

Fc = 0

B = length of line entered, including terminating null

C = cursor position on exit

A = character which caused end of input

Fc = 1

A = error code:

....DEHL/IXIY same

AFBC..../.... different

ld   a, 12

call_oz(OS_OUT)

call fetch

push iy

pop  hl

call_oz(GN_NLN)

call_oz(GN_SOP)

call_oz(GN_NLN)

ret

ld   a, init

push iy

pop  de

ld   b, 30

ld   c, 0

ld   hl, pos

call_oz(GN_SOP)

ld   l, 15

call_oz(GN_SIP)

ret  nc

cp   RC_SUSP

ret  nz

ld   b, 30

ld   a, init | 1

jr   sip

defm 1 & "3@" & 32 & 35

defm "Input: "

defb 0

%CO:C,15,56%

%H4%<SQUARE>%H4%

IN_ASPC, $B0

IN_AENT, $B1

IN_ATAB, $B2

IN_ADEL, $B3

IN_AESC, $B4

IN_AMEN, $B5

IN_AIDX, $B6

IN_AHEL, $B7

IN_ALFT, $F0

IN_ARGT, $F1

IN_ADWN, $F2

IN_AUP, $F3

%H4%SYMBOL%H4%

NUL

SOH

STX

ETX

EOT

ENQ

ACK

BEL

BS

HT

LF

VT

FF

CR

SO

SI

DLE

DC1

DC2

DC3

DC4

NAK

SYN

ETB

CAN

EM

SUB

ESC

FS

GS

RS

US

Delete line

Delete to end of line

Delete character under cursor, rightward

<ENTER>

Swap case

Delete word under cursor

Insert character

Insert/Overtype (see below for complications)

Start of line

End of line

Previous word

Next word

Next option , where input is limited all the option can

be cycled through using this command

Top of current page

Bottom of current page

Move up a screenful

Move down a screenful

Next column or TAB

Previous column or TAB

RC_BAD ($04), bad arguments

RC_WRAP ($0D), wrapping has occurred (only if A4 = 1)

RC_SUSP ($69), suspicion of suspension

RC_DRAW ($66), application screen needs redrawing

RC_QUIT ($67), kill request (e.g. from INDEX)

RC_ESC ($01), if escape detection is enabled

%CO:D,15,44%

%H4%<DIAMOND>%H4%

IN_DSPC, $C0

IN_DENT, $C1

IN_DTAB, $C2

IN_DDEL, $C3

IN_DESC, $C4

IN_DMEN, $C5

IN_DIDX, $C6

IN_DHEL, $C7

IN_DLFT, $F4

IN_DRGT, $F5

IN_DDWN, $F6

IN_DUP, $F7

%H4%KEYS%H4%

<>=

<>A

<>B

<>C

<>D

<>E

<>F

<>G

<>H

<>I

<>J

<>K

<>L

<>M

<>N

<>O

<>P

<>Q

<>R

<>S

<>T

<>U

<>V

<>W

<>X

<>Y

<>Z

<>[ or ESC

<>\

<>]

<>`

<>-

; get standard I/O definitions

; get error code definitions

; mode for GN_SIP (single line lock)

; clear screen

; fetch the line

; may need to check errors here...

; HL points at start of input buffer

; output a newline

; display input buffer

; terminate with another newline

; initial mode of GN_SIP

; DE points at input buffer

; max. buffer size

; cursor position at beginning

; input prompt text

; to standard output

; max. line width

; edit line buffer...

; return if no errors

; check for suspension

; return if some other error

; reset buffer length

; buffer contains data

; re-enter GN_SIP

; cursor at (0,3)

; prompt

; null-terminator

%CO:E,15,32%

%H4%<SHIFT>%H4%

-

IN_SENT, $D1

IN_STAB, $D2

IN_SDEL, $D3

IN_SESC, $D4 **

IN_SMEN, $D5

IN_SIDX, $D6

IN_SHEL, $D7

IN_SLFT, $F8

IN_SRGT, $F9

IN_SDWN, $FA

IN_SUP, $FB

%H4%DESCRIPTION%H4%

NULL

Start of header

Start of text

End of text

End of transmission

Enquiry

Acknowledge

Bell

Backspace

Horizontal tabulation

Line feed

Vertical tabulation

Form feed

Carriage return

Shift out

Shift in

Data link escape

Device control 1 (XON)

Device control 2

Device control 1 (XOFF)

Device control 4

Negative acknowledge

Synchronous idle

End of transmitted block

Cancel line

End of medium

Substitute (End of file)

Escape

File separator

Group separator

Record separator

Unit separator

%CO:F,17,20%